# ESP32 Series

Datasheet Version 5.2

2.4 GHz Wi-Fi + Bluetooth® + Bluetooth LE SoC

Including:

ESP32-D0WD-V3 ESP32-U4WDH

ESP32-S0WD – Not Recommended for New Designs (NRND) ESP32-D0WD – Not Recommended for New Designs (NRND) ESP32-D0WDQ6 – Not Recommended for New Designs (NRND) ESP32-D0WDQ6-V3 – Not Recommended for New Designs (NRND)

ESP32-D0WDR2-V3 – End of Life (EOL), upgraded to ESP32-D0WDRH2-V3

www.espressif.com

# Product Overview

ESP32 is a single 2.4 GHz Wi-Fi-and-Bluetooth combo chip designed with the TSMC low-power 40 nm technology. It is designed to achieve the best power and RF performance, showing robustness, versatility and reliability in a wide variety of applications and power scenarios.

For details on part numbers and ordering information, please refer to Section 1 ESP32 Series Comparison. For details on chip revisions, please refer to ESP32 Chip Revision v3.0 User Guide and

ESP32 Series SoC Errata.

Switch

Balun

The functional block diagram of the SoC is shown below.

In-Package Flash or PSRAM

SPI I2C I2S SDIO UART

TWAI®

ETH RMT PWM

Touch sensor DAC ADC

Timers

Bluetooth link controller

Bluetooth baseband

Wi-Fi MAC

Wi-Fi baseband

RF

receive

Clock generator

RF

transmit

Core and memory

2 (or 1) x Xtensa® 32-bit LX6 Microprocessors

Cryptographic hardware acceleration

SHA

RSA

ROM

SRAM

AES

RNG

RTC

PMU

ULP

coprocessor

Recovery memory

ESP32 Functional Block Diagram

## Features

Wi-Fi

- 802.11b/g/n

- 802.11n (2.4 GHz), up to 150 Mbps

- WMM

- TX/RX A-MPDU, RX A-MSDU

- Immediate Block ACK

- Defragmentation

- Automatic Beacon monitoring (hardware TSF)

- Four virtual Wi-Fi interfaces

- Simultaneous support for Infrastructure Station, SoftAP, and Promiscuous modes

Note that when ESP32 is in Station mode, performing a scan, the SoftAP channel will be changed.

- Antenna diversity

Bluetooth®

- Compliant with Bluetooth v4.2 BR/EDR and Bluetooth LE specifications

- Class-1, class-2 and class-3 transmitter without external power amplifier

- Enhanced Power Control

• +9 dBm transmitting power

- NZIF receiver with –94 dBm Bluetooth LE sensitivity

- Adaptive Frequency Hopping (AFH)

- Standard HCI based on SDIO/SPI/UART

- High-speed UART HCI, up to 4 Mbps

- Bluetooth 4.2 BR/EDR and Bluetooth LE dual mode controller

- Synchronous Connection-Oriented/Extended (SCO/eSCO)

- CVSD and SBC for audio codec

- Bluetooth Piconet and Scatternet

- Multi-connections in Classic Bluetooth and Bluetooth LE

- Simultaneous advertising and scanning

CPU and Memory

- Xtensa® single-/dual-core 32-bit LX6 microprocessor(s)

- CoreMark® score:

  - 1 core at 240 MHz: 539.98 CoreMark; 2.25 CoreMark/MHz

  - 2 cores at 240 MHz: 1079.96 CoreMark; 4.50 CoreMark/MHz

- 448 KB ROM

- 520 KB SRAM

- 16 KB SRAM in RTC

- QSPI supports multiple flash/SRAM chips

Clocks and Timers

- Internal 8 MHz oscillator with calibration

- Internal RC oscillator with calibration

- External 2 MHz ~ 60 MHz crystal oscillator (40 MHz only for Wi-Fi/Bluetooth functionality)

- External 32 kHz crystal oscillator for RTC with calibration

- Two timer groups, including 2 × 64-bit timers and 1 × main watchdog in each group

- One RTC timer

- RTC watchdog

Advanced Peripheral Interfaces

- 34 programmable GPIOs

  - Five strapping GPIOs

  - Six input-only GPIOs

  - Six GPIOs needed for in-package flash (ESP32-U4WDH) and in-package PSRAM (ESP32-D0WDRH2-V3)

- 12-bit SAR ADC up to 18 channels

- Two 8-bit DAC

- 10 touch sensors

- Four SPI interfaces

- Two I2S interfaces

- Two I2C interfaces

- Three UART interfaces

- One host (SD/eMMC/SDIO)

- One slave (SDIO/SPI)

- Pulse count controller

- Ethernet MAC interface with dedicated DMA and IEEE 1588 support

- TWAI®, compatible with ISO 11898-1 (CAN Specification 2.0)

- RMT (TX/RX)

- Motor PWM

- LED PWM up to 16 channels

Power Management

- Fine-resolution power control through a selection of clock frequency, duty cycle, Wi-Fi operating modes, and individual power control of internal components

- Five power modes designed for typical scenarios: Active, Modem-sleep, Light-sleep, Deep-sleep, Hibernation

- Power consumption in Deep-sleep mode is 10 µA

- Ultra-Low-Power (ULP) coprocessors

- RTC memory remains powered on in Deep-sleep mode

Security

- Secure boot

- Flash encryption

- 1024-bit OTP, up to 768-bit for customers

- Cryptographic hardware acceleration:

  - AES

  - Hash (SHA-2)

  - RSA

  - Random Number Generator (RNG)

## Applications

With low power consumption, ESP32 is an ideal choice for IoT devices in the following areas:

- Smart Home

- Industrial Automation

- Health Care

- Consumer Electronics

- Smart Agriculture

- POS Machines

- Service Robot

- Audio Devices

- Generic Low-power IoT Sensor Hubs

- Generic Low-power IoT Data Loggers

- Cameras for Video Streaming

- Speech Recognition

- Image Recognition

- SDIO Wi-Fi + Bluetooth Networking Card

Note:

Check the link or the QR code to make sure that you use the latest version of this document: https://www.espressif.com/documentation/esp32_datasheet_en.pdf

# Contents

Product Overview	2

Features	3

Applications	5

- ESP32 Series Comparison	11

  - Nomenclature	11

  - Comparison	11

- Pins	12

  - Pin Layout	12

  - Pin Overview	14

  - IO Pins	17

    - Restrictions for GPIOs and RTC_GPIOs	17

  - Analog Pins	17

  - Power Supply	17

    - Power Pins	17

    - Power Scheme	18

    - Chip Power-up and Reset	19

  - Pin Mapping Between Chip and Flash/PSRAM	20

- Boot Configurations	22

  - Chip Boot Mode Control	23

  - Internal LDO (VDD_SDIO) Voltage Control	24

  - U0TXD Printing Control	25

  - Timing Control of SDIO Slave	25

  - JTAG Signal Source Control	25

- Functional Description	26

  - CPU and Memory	26

    - CPU	26

    - Internal Memory	26

    - External Flash and RAM	27

    - Address Mapping Structure	27

    - Cache	29

  - System Clocks	29

    - CPU Clock	29

|  | 4.2.2 RTC Clock | 29 |
| --- | --- | --- |
|  | 4.2.3 Audio PLL Clock | 30 |
| 4.3 | RTC and Low-power Management | 30 |
|  | 4.3.1 Power Management Unit (PMU) | 30 |
|  | 4.3.2 Ultra-Low-Power Coprocessor | 31 |
| 4.4 | Timers and Watchdogs | 31 |
|  | 4.4.1 General Purpose Timers | 31 |
|  | 4.4.2 Watchdog Timers | 31 |
| 4.5 | Cryptographic Hardware Accelerators | 32 |
| 4.6 | Radio and Wi-Fi | 32 |
|  | 4.6.1 2.4 GHz Receiver | 32 |
|  | 4.6.2 2.4 GHz Transmitter | 33 |
|  | 4.6.3 Clock Generator | 33 |
|  | 4.6.4 Wi-Fi Radio and Baseband | 33 |
|  | 4.6.5 Wi-Fi MAC | 34 |
| 4.7 | Bluetooth | 34 |
|  | 4.7.1 Bluetooth Radio and Baseband | 34 |
|  | 4.7.2 Bluetooth Interface | 34 |
|  | 4.7.3 Bluetooth Stack | 35 |
|  | 4.7.4 Bluetooth Link Controller | 35 |
| 4.8 | Digital Peripherals | 37 |
|  | 4.8.1 General Purpose Input / Output Interface (GPIO) | 37 |
|  | 4.8.2 Serial Peripheral Interface (SPI) | 37 |
|  | 4.8.3 Universal Asynchronous Receiver Transmitter (UART) | 37 |
|  | 4.8.4 I2C Interface | 38 |
|  | 4.8.5 I2S Interface | 39 |
|  | 4.8.6 Remote Control Peripheral | 39 |
|  | 4.8.7 Pulse Counter Controller (PCNT) | 39 |
|  | 4.8.8 LED PWM Controller | 40 |
|  | 4.8.9 Motor Control PWM | 40 |
|  | 4.8.10 SD/SDIO/MMC Host Controller | 41 |
|  | 4.8.11 SDIO/SPI Slave Controller | 42 |
|  | 4.8.12 TWAI® Controller | 43 |
|  | 4.8.13 Ethernet MAC Interface | 43 |
| 4.9 | Analog Peripherals | 44 |
|  | 4.9.1 Analog-to-Digital Converter (ADC) | 44 |
|  | 4.9.2 Digital-to-Analog Converter (DAC) | 45 |
|  | 4.9.3 Touch Sensor | 45 |
| 4.10 | Peripheral Pin Configurations | 47 |
| 5 | Electrical Characteristics | 52 |
| 5.1 | Absolute Maximum Ratings | 52 |
| 5.2 | Recommended Power Supply Characteristics | 52 |
| 5.3 | DC Characteristics (3.3 V, 25 °C) | 53 |
| 5.4 | RF Current Consumption in Active Mode | 53 |
| 5.5 | Reliability | 54 |

  - Wi-Fi Radio	54

  - Bluetooth Radio	55

    - Receiver –Basic Data Rate	55

    - Transmitter –Basic Data Rate	55

    - Receiver –Enhanced Data Rate	56

    - Transmitter –Enhanced Data Rate	56

  - Bluetooth LE Radio	57

    - Receiver	57

    - Transmitter	59

- Packaging	60

Related Documentation and Resources	61

Appendix A –ESP32 Pin Lists	62

  - Notes on ESP32 Pin Lists	62

  - GPIO_Matrix	64

  - Ethernet_MAC	69

  - IO_MUX	69

Revision History	71

# List of Tables

  - ESP32 Series Comparison	11

  - Pin Overview	14

  - Analog Pins	17

  - Power Pins	18

  - Description of Timing Parameters for Power-up and Reset	19

  - Pin-to-Pin Mapping Between Chip and In-Package Flash/PSRAM	20

  - Pin-to-Pin Mapping Between Chip and Off-Package Flash/PSRAM	20

  - Default Configuration of Strapping Pins	22

  - Description of Timing Parameters for the Strapping Pins	23

  - Chip Boot Mode Control	23

  - U0TXD Printing Control	25

  - Timing Control of SDIO Slave	25

  - Memory and Peripheral Mapping	28

  - Power Consumption by Power Modes	30

  - ADC Characteristics	44

  - ADC Calibration Results	45

  - Capacitive-Sensing GPIOs Available on ESP32	46

  - Peripheral Pin Configurations	47

  - Absolute Maximum Ratings	52

  - Recommended Power Supply Characteristics	52

  - DC Characteristics (3.3 V, 25 °C)	53

  - Current Consumption Depending on RF Modes	53

  - Reliability Qualifications	54

  - Wi-Fi Radio Characteristics	54

  - Receiver Characteristics –Basic Data Rate	55

  - Transmitter Characteristics –Basic Data Rate	55

  - Receiver Characteristics –Enhanced Data Rate	56

  - Transmitter Characteristics –Enhanced Data Rate	57

  - Receiver Characteristics –Bluetooth LE	57

  - Transmitter Characteristics –Bluetooth LE	59

  - Notes on ESP32 Pin Lists	62

  - GPIO_Matrix	64

  - Ethernet_MAC	69

# List of Figures

  - ESP32 Series Nomenclature	11

  - ESP32 Pin Layout (QFN 6*6, Top View)	12

  - ESP32 Pin Layout (QFN 5*5, Top View)	13

  - ESP32 Power Scheme	18

  - Visualization of Timing Parameters for Power-up and Reset	19

  - Visualization of Timing Parameters for the Strapping Pins	23

  - Chip Boot Flow	24

  - Address Mapping Structure	27

  - QFN48 (6×6 mm) Package	60

  - QFN48 (5×5 mm) Package	60

# ESP32 Series Comparison

## Nomenclature

ESP32

D	0	WD	R2	H	Q6	V3

Chip revision v3.0 or newer Package

Q6: QFN 6*6 N/A: QFN 5*5

High temperature

In-package PSRAM

R2: 2 MB PSRAM

Connection

WD: Wi-Fi b/g/n + Bluetooth/Bluetooth LE dual mode

In-package flash

0: No in-package flash 2: 2 MB flash

4: 4 MB flash

Core

D/U: Dual core S: Single core

Chip Series

Figure 1-1. ESP32 Series Nomenclature

## Comparison

Table 1-1. ESP32 Series Comparison

| Part Number1 | Core | Chip Revision2 | In-Package Flash/PSRAM | Package | VDD_SDIO Voltage |
| --- | --- | --- | --- | --- | --- |
| ESP32-D0WD-V3 | Dual core | v3.0/v3.14 | — | QFN 5*5 | 1.8 V/3.3 V |
| ESP32-D0WDR2-V3 (EOL) Upgraded to ESP32-D0WDRH2-V3 7 | Dual core | v3.0/v3.14 | 2 MB PSRAM | QFN 5*5 | 3.3 V |
| ESP32-U4WDH | Dual core3 | v3.0/v3.14 | 4 MB flash6 | QFN 5*5 | 3.3 V |
| ESP32-D0WDQ6-V3 (NRND) | Dual core | v3.0/v3.14 | — | QFN 6*6 | 1.8 V/3.3 V |
| ESP32-D0WD (NRND) | Dual core | v1.0/v1.15 | — | QFN 5*5 | 1.8 V/3.3 V |
| ESP32-D0WDQ6 (NRND) | Dual core | v1.0/v1.15 | — | QFN 6*6 | 1.8 V/3.3 V |
| ESP32-S0WD (NRND) | Single core | v1.0/v1.15 | — | QFN 5*5 | 1.8 V/3.3 V |

1 All above chips support Wi-Fi b/g/n + Bluetooth/Bluetooth LE Dual Mode connection. For details on chip marking and packing, see Section 6 Packaging.

2 Differences between ESP32 chip revisions and how to distinguish them are described in ESP32 Series SoC Errata.

3 ESP32-U4WDH will be produced as dual-core instead of single core. See PCN-2021-021 for details.

4 The chips will be produced with chip revision v3.1 inside. See PCN20220901 for details.

5 The chips will be produced with chip revision v1.1 inside. See PCN20220901 for details.

6 The in-package flash supports:

- More than 100,000 program/erase cycles

- More than 20 years data retention time

7 ESP32-D0WDR2-V3 is end of life and upgraded to ESP32-D0WDRH2-V3. See PCN20251001 for details.

# Pins

CAP1 CAP2 VDDA XTAL_P XTAL_N VDDA GPIO21 UOTXD UORXD GPIO22 GPIO19

VDD3P3_CPU

Pin Layout

48

47

46

45

44

43

42

41

40

39

38

37

VDDA	1

LNA_IN	2

VDD3P3	3

VDD3P3	4

SENSOR_VP	5

SENSOR_CAPP	6

SENSOR_CAPN	7

SENSOR_VN	8

CHIP_PU	9

VDET_1	10

36  GPIO23

ESP32 49 GND

35  GPIO18

34	GPIO5

33  SD_DATA_1

32	SD_DATA_O

31	SD_CLK

30  SD_CMD

29	SD_DATA_3

28	SD_DATA_2

27	GPIO17

VDET_2	11

32K_XP	12

26	VDD_SDIO

13

14

15

16

17

18

19

20

21

22

23

24

25	GPIO16

32K_XN GPIO25 GPIO26 GPIO27 MTMS MTDI

VDD3P3_RTC

MTCK MTDO GPIO2 GPIOO GPIO4

Figure 2-1. ESP32 Pin Layout (QFN 6*6, Top View)

48

47

46

45

44

43

42

41

40

39

CAP1 CAP2 VDDA XTAL_P XTAL_N VDDA GPIO21 UOTXD UORXD

GPIO22

VDDA LNA_IN VDD3P3 VDD3P3 SENSOR_VP SENSOR_CAPP SENSOR_CAPN SENSOR_VN CHIP_PU VDET_1 VDET_2 32K_XP

32K_XN

15

16

17

18

19

20

21

22

23

24

GPIO25

1

2

3

4

5

6

7

8

9

10

11

12

13

14

38  GPIO19

ESP32 49 GND

37	VDD3P3_CPU

36  GPIO23

35  GPIO18

34  GPIO5

33  SD_DATA_1

32  SD_DATA_O

31	SD_CLK

30  SD_CMD

29  SD_DATA_3

28	SD_DATA_2

27	GPIO17

26  VDD_SDIO

25  GPIO16

GPIO26 GPIO27 MTMS MTDI

VDD3P3_RTC

MTCK MTDO GPIO2 GPIOO GPIO4

Figure 2-2. ESP32 Pin Layout (QFN 5*5, Top View)

2  Pins

Espressif Systems

14

Submit Documentation Feedback

ESP32 Series Datasheet v5.2

Pin Overview

Table 2-1. Pin Overview

| Name | No. | Type | Function |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analog |  |  |  |  |  |  |  |  |
| VDDA | 1 | P | Analog power supply (2.3 V ∼ 3.6 V) |  |  |  |  |  |
| LNA_IN | 2 | I/O | RF input and output |  |  |  |  |  |
| VDD3P3 | 3 | P | Analog power supply (2.3 V ∼ 3.6 V) |  |  |  |  |  |
| VDD3P3 | 4 | P | Analog power supply (2.3 V ∼ 3.6 V) |  |  |  |  |  |
| VDD3P3_RTC |  |  |  |  |  |  |  |  |
| SENSOR_VP | 5 | I | GPIO36, | ADC1_CH0, | RTC_GPIO0 |  |  |  |
| SENSOR_CAPP | 6 | I | GPIO37, | ADC1_CH1, | RTC_GPIO1 |  |  |  |
| SENSOR_CAPN | 7 | I | GPIO38, | ADC1_CH2, | RTC_GPIO2 |  |  |  |
| SENSOR_VN | 8 | I | GPIO39, | ADC1_CH3, | RTC_GPIO3 |  |  |  |
| CHIP_PU | 9 | I | High: On; enables the chip Low: Off; the chip shuts down Note: Do not leave the CHIP_PU pin floating. |  |  |  |  |  |
| VDET_1 | 10 | I | GPIO34, | ADC1_CH6, | RTC_GPIO4 |  |  |  |
| VDET_2 | 11 | I | GPIO35, | ADC1_CH7, | RTC_GPIO5 |  |  |  |
| 32K_XP | 12 | I/O | GPIO32, | ADC1_CH4, | RTC_GPIO9, | TOUCH9, | 32K_XP (32.768 kHz crystal oscillator input) |  |
| 32K_XN | 13 | I/O | GPIO33, | ADC1_CH5, | RTC_GPIO8, | TOUCH8, | 32K_XN (32.768 kHz crystal oscillator output) |  |
| GPIO25 | 14 | I/O | GPIO25, | ADC2_CH8, | RTC_GPIO6, | DAC_1, | EMAC_RXD0 |  |
| GPIO26 | 15 | I/O | GPIO26, | ADC2_CH9, | RTC_GPIO7, | DAC_2, | EMAC_RXD1 |  |
| GPIO27 | 16 | I/O | GPIO27, | ADC2_CH7, | RTC_GPIO17, | TOUCH7, | EMAC_RX_DV |  |
| MTMS | 17 | I/O | GPIO14, | ADC2_CH6, | RTC_GPIO16, | TOUCH6, | EMAC_TXD2, HSPICLK, HS2_CLK, SD_CLK, | MTMS |
| MTDI | 18 | I/O | GPIO12, | ADC2_CH5, | RTC_GPIO15, | TOUCH5, | EMAC_TXD3, HSPIQ, HS2_DATA2, SD_DATA2, | MTDI |
| VDD3P3_RTC | 19 | P | Input power supply for RTC IO (2.3 V ∼ 3.6 V) |  |  |  |  |  |
| MTCK | 20 | I/O | GPIO13, | ADC2_CH4, | RTC_GPIO14, | TOUCH4, | EMAC_RX_ER, HSPID, HS2_DATA3, SD_DATA3, | MTCK |
| MTDO | 21 | I/O | GPIO15, | ADC2_CH3, | RTC_GPIO13, | TOUCH3, | EMAC_RXD3, HSPICS0, HS2_CMD, SD_CMD, | MTDO |

2  Pins

Espressif Systems

15

Submit Documentation Feedback

ESP32 Series Datasheet v5.2

| Name | No. | Type | Function |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPIO2 | 22 | I/O | GPIO2, | ADC2_CH2, | RTC_GPIO12, | TOUCH2, |  | HSPIWP, | HS2_DATA0, | SD_DATA0 |
| GPIO0 | 23 | I/O | GPIO0, | ADC2_CH1, | RTC_GPIO11, | TOUCH1, EMAC_TX_CLK, |  | CLK_OUT1, |  |  |
| GPIO4 | 24 | I/O | GPIO4, | ADC2_CH0, | RTC_GPIO10, | TOUCH0, EMAC_TX_ER, |  | HSPIHD, | HS2_DATA1, | SD_DATA1 |
| VDD_SDIO |  |  |  |  |  |  |  |  |  |  |
| GPIO16 | 25 | I/O | GPIO16, | HS1_DATA4, | U2RXD, | EMAC_CLK_OUT |  |  |  |  |
| VDD_SDIO | 26 | P | Output power supply: 1.8 V or the same voltage as VDD3P3_RTC |  |  |  |  |  |  |  |
| GPIO17 | 27 | I/O | GPIO17, | HS1_DATA5, | U2TXD, | EMAC_CLK_OUT_180 |  |  |  |  |
| SD_DATA_2 | 28 | I/O | GPIO9, | HS1_DATA2, | U1RXD, | SD_DATA2, | SPIHD |  |  |  |
| SD_DATA_3 | 29 | I/O | GPIO10, | HS1_DATA3, | U1TXD, | SD_DATA3, | SPIWP |  |  |  |
| SD_CMD | 30 | I/O | GPIO11, | HS1_CMD, | U1RTS, | SD_CMD, | SPICS0 |  |  |  |
| SD_CLK | 31 | I/O | GPIO6, | HS1_CLK, | U1CTS, | SD_CLK, | SPICLK |  |  |  |
| SD_DATA_0 | 32 | I/O | GPIO7, | HS1_DATA0, | U2RTS, | SD_DATA0, | SPIQ |  |  |  |
| SD_DATA_1 | 33 | I/O | GPIO8, | HS1_DATA1, | U2CTS, | SD_DATA1, | SPID |  |  |  |
| VDD3P3_CPU |  |  |  |  |  |  |  |  |  |  |
| GPIO5 | 34 | I/O | GPIO5, | HS1_DATA6, | VSPICS0, | EMAC_RX_CLK |  |  |  |  |
| GPIO18 | 35 | I/O | GPIO18, | HS1_DATA7, | VSPICLK |  |  |  |  |  |
| GPIO23 | 36 | I/O | GPIO23, | HS1_STROBE, | VSPID |  |  |  |  |  |
| VDD3P3_CPU | 37 | P | Input power supply for CPU IO (1.8 V ∼ 3.6 V) |  |  |  |  |  |  |  |
| GPIO19 | 38 | I/O | GPIO19, | U0CTS, | VSPIQ, | EMAC_TXD0 |  |  |  |  |
| GPIO22 | 39 | I/O | GPIO22, | U0RTS, | VSPIWP, | EMAC_TXD1 |  |  |  |  |
| U0RXD | 40 | I/O | GPIO3, | U0RXD, | CLK_OUT2 |  |  |  |  |  |
| U0TXD | 41 | I/O | GPIO1, | U0TXD, | CLK_OUT3, | EMAC_RXD2 |  |  |  |  |
| GPIO21 | 42 | I/O | GPIO21, |  | VSPIHD, | EMAC_TX_EN |  |  |  |  |
| Analog |  |  |  |  |  |  |  |  |  |  |
| VDDA | 43 | P | Analog power supply (2.3 V ∼ 3.6 V) |  |  |  |  |  |  |  |
| XTAL_N | 44 | O | External crystal output |  |  |  |  |  |  |  |
| XTAL_P | 45 | I | External crystal input |  |  |  |  |  |  |  |
| VDDA | 46 | P | Analog power supply (2.3 V ∼ 3.6 V) |  |  |  |  |  |  |  |
| CAP2 | 47 | I | Connects to a 3.3 nF (10%) capacitor and 20 kΩ resistor in parallel to CAP1 |  |  |  |  |  |  |  |

| Name | No. | Type | Function |
| --- | --- | --- | --- |
| CAP1 | 48 | I | Connects to a 10 nF series capacitor to ground |
| GND | 49 | P | Ground |

Notes for Table 2-1 Pin Overview:

- Function names:

CLK_OUT…	clock output

SPICLK HSPICLK VSPICLK





SPI clock signal

HS…_CLK	SDIO Master clock signal

SD_CLK	SDIO Slave clock signal

EMAC_TX_CLK EMAC_RX_CLK

U…_RTS U…_CTS U…_RXD U…_TXD MTMS

} EMAC clock signal

} UART0/1/2 hardware flow control signals

}  UART0/1/2 receive/transmit signals





MTDI MTCK

MTDO 

JTAG interface signals

2  Pins

Espressif Systems

16

Submit Documentation Feedback

ESP32 Series Datasheet v5.2

GPIO…	General-purpose input/output with signals routed via the GPIO matrix. For

more details on the GPIO matrix, see ESP32 Technical Reference Manual > Chapter IO MUX and GPIO Matrix。

- Regarding highlighted cells, see Section 2.3.1 Restrictions for GPIOs and RTC_GPIOs.

- For a quick reference guide to using the IO_MUX, Ethernet MAC, and GPIO Matrix pins of ESP32, please refer to Appendix ESP32 Pin Lists.

## IO Pins

### Restrictions for GPIOs and RTC_GPIOs

All IO pins of the ESP32 have GPIO and some have RTC_GPIO pin functions. However, these IO pins are multifunctional and can be configured for different purposes based on the requirements. Some IOs have restrictions for usage. It is essential to consider their multiplexed nature and the limitations when using these IO pins.

In Table 2-1 Pin Overview some pin functions are highlighted, specically:

      - GPIO – Input only pins, output is not supported due to lack of pull-up/pull-down resistors.

      - GPIO  – allocated for communication with in-package flash/PSRAM and NOT recommended for other uses. For details, see Section 2.6 Pin Mapping Between Chip and Flash/PSRAM.

      - GPIO – have one of the following important functions:

        - Strapping pins – need to be at certain logic levels at startup. See Section 3 Boot Configurations.

        - JTAG interface – often used for debugging.

        - UART interface – often used for debugging.

See also Appendix A.1 – Notes on ESP32 Pin Lists.

## Analog Pins

Table 2-2. Analog Pins

| Pin No. | Pin Name | Pin Type | Pin Function |
| --- | --- | --- | --- |
| 2 | LNA_IN | I/O | Low Noise Amplifier (LNA) input signal, Power Amplifier (PA) output signal |
| 9 | CHIP_PU | I | High: on, enables the chip (Powered up). Low: off, the chip powers off (powered down). Note: Do not leave the CHIP_PU pin floating. |
| 44 | XTAL_N | — | External clock input/output connected to chip’s crystal or oscillator. P/N means differential clock positive/negative. |
| 45 | XTAL_P | — |  |

## Power Supply

### Power Pins

ESP32’s digital pins are divided into three different power domains:

      - VDD3P3_RTC

      - VDD3P3_CPU

      - VDD_SDIO

Table 2-3. Power Pins

| Pin No. | Pin Name | Direction | Power Supply |  |
| --- | --- | --- | --- | --- |
|  |  |  | Power Domain / Other | IO Pins |
| 1 | VDDA | Input | Analog power domain |  |
| 3 | VDD3P3 | Input | Analog power domain |  |
| 4 | VDD3P3 | Input | Analog power domain |  |
| 19 | VDD3P3_RTC1 | Input | RTC and part of Digital power domains | RTC IO |
| 26 | VDD3P3_SDIO2 | Input/Output | Analog power domain |  |
| 37 | VDD3P3_CPU3 | Input | Digital power domain | Digital IO |
| 43 | VDDA | Input | Analog power domain |  |
| 46 | VDDA | Input | Analog power domain |  |
| 49 | GND | – | External ground connection |  |

1 VDD3P3_RTC is also the input power supply for RTC and CPU.

2 VDD_SDIO connects to the output of an internal LDO whose input is VDD3P3_RTC. When VDD_SDIO is connected to the same PCB net together with VDD3P3_RTC, the internal LDO is disabled automatically.

3 VDD3P3_CPU is also the input power supply for CPU.

### Power Scheme

The power scheme is shown in Figure 2-3 ESP32 Power Scheme.

VDD3P3_RTC	VDD3P3_CPU

1.8 V

LDO

R = 6 ▲

LDO	1.1 V

LDO	1.1 V

O

SDIO

RTC

CPU

Domain

Domain

Domain

VDD_SDI

3.3 V/1.8 V

Figure 2-3. ESP32 Power Scheme

The internal LDO can be configured as having 1.8 V, or the same voltage as VDD3P3_RTC. It can be powered off via software to minimize the current of flash/SRAM during the Deep-sleep mode.

### Chip Power-up and Reset

Once the power is supplied to the chip, its power rails need a short time to stabilize. After that, CHIP_PU – the pin used for power-up and reset – is pulled high to activate the chip. For information on CHIP_PU as well as power-up and reset timing, see Figure 2-4 and Table 2-4.

VDD3P3_RTC Min

VIL_nRST

tSTBL	tRST

VDD

CHIP_PU

Figure 2-4. Visualization of Timing Parameters for Power-up and Reset

Table 2-4. Description of Timing Parameters for Power-up and Reset

| Parameter | Description | Min (µs) |
| --- | --- | --- |
| tSTBL | Time reserved for the 3.3 V rails to stabilize before the CHIP_PU pin is pulled high to activate the chip | 50 |
| tRST | Time reserved for CHIP_PU to stay below VIL_nRST to reset the chip (see Table 5-3) | 50 |

- In scenarios where ESP32 is powered up and down repeatedly by switching the power rails, while there is a large capacitor on the VDD33 rail and CHIP_PU and VDD33 are connected, simply switching off the CHIP_PU power rail and immediately switching it back on may cause an incomplete power discharge cycle and failure to reset the chip adequately.

An additional discharge circuit may be required to accelerate the discharge of the large capacitor on rail VDD33, which will ensure proper power-on-reset when the ESP32 is powered up again.

- When a battery is used as the power supply for the ESP32 series of chips and modules, a supply voltage supervisor is recommended, so that a boot failure due to low voltage is avoided. Users are recommended to pull CHIP_PU low if the power supply for ESP32 is below 2.3 V.

Notes on power supply:

- The operating voltage of ESP32 ranges from 2.3 V to 3.6 V. When using a single-power supply, the recommended voltage of the power supply is 3.3 V, and its recommended output current is 500 mA or more.

- PSRAM and flash both are powered by VDD_SDIO. If the chip has an in-package flash, the voltage of VDD_SDIO is determined by the operating voltage of the in-package flash. If the chip also connects to an external PSRAM, the operating voltage of external PSRAM must match that of the in-package flash. This also applies if the chip has an in-package PSRAM but also connects to an external flash.

- When VDD_SDIO 1.8 V is used as the power supply for external flash/PSRAM, a 2 kΩ grounding resistor should be added to VDD_SDIO. For the circuit design, please refer to

ESP32 Hardware Design Guidelines.

- When the three digital power supplies are used to drive peripherals, e.g., 3.3 V flash, they should comply with the peripherals’ specifications.

## Pin Mapping Between Chip and Flash/PSRAM

Table 2-5 lists the pin-to-pin mapping between the chip and the in-package flash/PSRAM. The chip pins listed here are not recommended for other usage.

For the data port connection between ESP32 and off-package flash/PSRAM please refer to Table 2-6.

Table 2-5. Pin-to-Pin Mapping Between Chip and In-Package Flash/PSRAM

| ESP32-U4WDH | In-Package Flash (4 MB) |
| --- | --- |
| SD_DATA_1 | IO0/DI |
| GPIO17 | IO1/DO |
| SD_DATA_0 | IO2/WP# |
| SD_CMD | IO3/HOLD# |
| SD_CLK | CLK |
| GPIO16 | CS# |
| GND | VSS |
| VDD_SDIO1 | VDD |
| ESP32-D0WDRH2-V3 | In-Package PSRAM (2 MB) |
| SD_DATA_1 | SIO0/SI |
| SD_DATA_0 | SIO1/SO |
| SD_DATA_3 | SIO2 |
| SD_DATA_2 | SIO3 |
| SD_CLK | SCLK |
| GPIO162 | CE# |
| GND | VSS |
| VDD_SDIO1 | VDD |

Table 2-6. Pin-to-Pin Mapping Between Chip and Off-Package Flash/PSRAM

| Chip Pin | Off-Package Flash |
| --- | --- |
| SD_DATA_1/SPID | IO0/DI |
| SD_DATA_0/SPIQ | IO1/DO |
| SD_DATA_3/SPIWP | IO2/WP# |
| SD_DATA_2/SPIHD | IO3/HOLD# |
| SD_CLK | CLK |
| SD_CMD | CS# |
| GND | VSS |
| VDD_SDIO | VDD |

Cont’d on next page

Table 2-6 – cont’d from previous page

| Chip Pin | Off-Package PSRAM |
| --- | --- |
| Chip Pin | Off-Package PSRAM |
| SD_DATA_1 | SIO0/SI |
| SD_DATA_0 | SIO1/SO |
| SD_DATA_3 | SIO2 |
| SD_DATA_2 | SIO3 |
| SD_CLK/GPIO173 | SCLK |
| GPIO162 | CE# |
| GND | VSS |
| VDD_SDIO | VDD |

Note:

- As the in-package flash (ESP32-U4WDH) and the in-package PSRAM (ESP32-D0WDRH2-V3) operate at 3.3 V, VDD_SDIO must be powered by VDD3P3_RTC via a 6 Ω resistor. See Figure 2-3 ESP32 Power Scheme.

- If GPIO16 is used to connect to PSRAM’s CE# signal, please add a pull-up resistor at the GPIO16 pin. See

ESP32-WROVER-E Datasheet > Figure Schematics of ESP32-WROVER-E.

- SD_CLK and GPIO17 pins are available to connect to the SCLK signal of external PSRAM.

  - If SD_CLK pin is selected, one GPIO (i.e., GPIO17) will be saved. The saved GPIO can be used for other purposes. This connection has passed internal tests, but relevant certification has not been completed.

  - Or GPIO17 pin is used to connect to the SCLK signal. This connection has passed relevant certification, see certificates for ESP32-WROVER-E.

Please select the proper pin for your specific applications.

# Boot Configurations

The chip allows for configuring the following boot parameters through strapping pins and eFuse bits at power-up or a hardware reset, without microcontroller interaction.

- Chip boot mode

  - Strapping pin: GPIO0 and GPIO2

- Internal LDO (VDD_SDIO) Voltage

  - Strapping pin: MTDI

  - eFuse bit: EFUSE_SDIO_FORCE and EFUSE_SDIO_TIEH

- U0TXD printing

  - Strapping pin: MTDO

- Timing of SDIO Slave

  - Strapping pin: MTDO and GPIO5

- JTAG signal source

  - eFuse bit: EFUSE_DISABLE_JTAG

The default values of all the above eFuse bits are 0, which means that they are not burnt. Given that eFuse is one-time programmable, once an eFuse bit is programmed to 1, it can never be reverted to 0. For how to program eFuse bits, please refer to ESP32 Technical Reference Manual > Chapter eFuse Controller.

The default values of the strapping pins, namely the logic levels, are determined by pins’internal weak pull-up/pull-down resistors at reset if the pins are not connected to any circuit, or connected to an external high-impedance circuit.

Table 3-1. Default Configuration of Strapping Pins

| Strapping Pin | Default Configuration | Bit Value |
| --- | --- | --- |
| GPIO0 | Pull-up | 1 |
| GPIO2 | Pull-down | 0 |
| MTDI | Pull-down | 0 |
| MTDO | Pull-up | 1 |
| GPIO5 | Pull-up | 1 |

To change the bit values, the strapping pins should be connected to external pull-down/pull-up resistances. If the ESP32 is used as a device by a host MCU, the strapping pin voltage levels can also be controlled by the host MCU.

All strapping pins have latches. At system reset, the latches sample the bit values of their respective strapping pins and store them until the chip is powered down or shut down. The states of latches cannot be changed in any other way. It makes the strapping pin values available during the entire chip operation, and the pins are freed up to be used as regular IO pins after reset.

The timing of signals connected to the strapping pins should adhere to the setup time and hold time

specifications in Table 3-2 and Figure 3-1.

Table 3-2. Description of Timing Parameters for the Strapping Pins

| Parameter | Description | Min (ms) |
| --- | --- | --- |
| tSU | Setup time is the time reserved for the power rails to stabilize be- fore the CHIP_PU pin is pulled high to activate the chip. | 0 |
| tH | Hold time is the time reserved for the chip to read the strapping pin values after CHIP_PU is already high and before these pins start operating as regular IO pins. | 1 |

VIH_nRST

VIH

tSU	tH

CHIP_PU

Strapping pin

Figure 3-1. Visualization of Timing Parameters for the Strapping Pins

## Chip Boot Mode Control

GPIO0 and GPIO2 control the boot mode after the reset is released. See Table 3-3 Chip Boot Mode Control.

Table 3-3. Chip Boot Mode Control

| Boot Mode | GPIO0 | GPIO2 |
| --- | --- | --- |
| SPI Boot Mode | 1 | Any value |
| Joint Download Boot Mode 2 | 0 | 0 |

1 Bold marks the default value and configuration.

2 Joint Download Boot mode supports the following download methods:

- SDIO Download Boot

- UART Download Boot

In Joint Download Boot mode, the detailed boot flow of the chip is put below 3-2.

Figure 3-2. Chip Boot Flow

uart_download_dis controls boot mode behaviors:

It permanently disables Download Boot mode when uart_download_dis is set to 1 (valid only for ESP32 chip revisions v3.0 and higher).

## Internal LDO (VDD_SDIO) Voltage Control

The required VDD_SPI voltage for the chips of the ESP32 Series can be found in Table 1-1 Comparison. MTDI is used to select the VDD_SDIO power supply voltage at reset:

- MTDI = 0 (by default), VDD_SDIO pin is powered directly from VDD3P3_RTC. Typically this voltage is 3.3

V. For more information, see Section 2.5.2 Power Scheme.

- MTDI = 1, VDD_SDIO pin is powered from internal 1.8 V LDO.

This functionality can be overridden by setting EFUSE_SDIO_FORCE to 1, in which case the EFUSE_SDIO_TIEH determines the VDD_SDIO voltage:

- EFUSE_SDIO_TIEH = 0, VDD_SDIO connects to 1.8 V LDO.

- EFUSE_SDIO_TIEH = 1, VDD_SDIO connects to VDD3P3_RTC.

## U0TXD Printing Control

During booting, the strapping pin MTDO can be used to control the U0TXD Printing, as Table 3-4 shows.

Table 3-4. U0TXD Printing Control

| U0TXD Printing Control | MTDO |
| --- | --- |
| Enabled 1 | 1 |
| Disabled | 0 |

1 Bold marks the default value and configuration.

## Timing Control of SDIO Slave

The strapping pin MTDO and GPIO5 can be used to control the timing of SDIO slave, see Table 3-5 Timing Control of SDIO Slave.

Table 3-5. Timing Control of SDIO Slave

| Edge behavior | MTDO | GPIO5 |
| --- | --- | --- |
| Falling edge sampling, falling edge output | 0 | 0 |
| Falling edge sampling, rising edge output | 0 | 1 |
| Rising edge sampling, falling edge output | 1 | 0 |
| Rising edge sampling, rising edge output | 1 | 1 |

1 Bold marks the default value and configuration.

## JTAG Signal Source Control

If EFUSE_DISABLE_JTAG is set to 1, the source of JTAG signals can be disabled.

# Functional Description

## CPU and Memory

### CPU

ESP32 contains one or two low-power Xtensa® 32-bit LX6 microprocessor(s) with the following features:

      - 7-stage pipeline to support the clock frequency of up to 240 MHz (160 MHz for ESP32-S0WD (NRND))

      - 16/24-bit Instruction Set provides high code-density

      - Support for Floating Point Unit

      - Support for DSP instructions, such as a 32-bit multiplier, a 32-bit divider, and a 40-bit MAC

      - Support for 32 interrupt vectors from about 70 interrupt sources The single-/dual-CPU interfaces include:

      - Xtensa RAM/ROM Interface for instructions and data

      - Xtensa Local Memory Interface for fast peripheral register access

      - External and internal interrupt sources

      - JTAG for debugging

For information about the Xtensa® Instruction Set Architecture, please refer to

Xtensa® Instruction Set Architecture (ISA) Summary.

### Internal Memory

ESP32’s internal memory includes:

      - 448 KB of ROM for booting and core functions

      - 520 KB of on-chip SRAM for data and instructions

      - 8 KB of SRAM in RTC, which is called RTC FAST Memory and can be used for data storage; it is accessed by the main CPU during RTC Boot from the Deep-sleep mode.

      - 8 KB of SRAM in RTC, which is called RTC SLOW Memory and can be accessed by the ULP coprocessor during the Deep-sleep mode.

      - 1 Kbit of eFuse: 256 bits are used for the system (MAC address and chip configuration) and the remaining 768 bits are reserved for customer applications, including flash-encryption and chip-ID.

      - In-package flash or PSRAM

Note:

Products in the ESP32 series differ from each other, in terms of their support for in-package flash or PSRAM and the size of them. For details, please refer to Section 1 ESP32 Series Comparison.

### External Flash and RAM

ESP32 supports multiple external QSPI flash and external RAM (SRAM) chips. More details can be found in ESP32 Technical Reference Manual > Chapter SPI Controller. ESP32 also supports hardware encryption/decryption based on AES to protect developers’ programs and data in flash.

ESP32 can access the external QSPI flash and SRAM through high-speed caches.

      - Up to 16 MB of external flash can be mapped into CPU instruction memory space and read-only memory space simultaneously.

        - When external flash is mapped into CPU instruction memory space, up to 11 MB + 248 KB can be mapped at a time. Note that if more than 3 MB + 248 KB are mapped, cache performance will be reduced due to speculative reads by the CPU.

        - When external flash is mapped into read-only data memory space, up to 4 MB can be mapped at a time. 8-bit, 16-bit and 32-bit reads are supported.

      - External RAM can be mapped into CPU data memory space. SRAM up to 8 MB is supported and up to 4 MB can be mapped at a time. 8-bit, 16-bit and 32-bit reads and writes are supported.

Note:

After ESP32 is initialized, firmware can customize the mapping of external RAM or flash into the CPU address space.

### Address Mapping Structure

The structure of address mapping is shown in Figure 4-1. The memory and peripheral mapping is shown in Table 4-1.

Figure 4-1. Address Mapping Structure

Table 4-1. Memory and Peripheral Mapping

| Category | Target | Start Address | End Address | Size |
| --- | --- | --- | --- | --- |
| Embedded Memory | Internal ROM 0 | 0×4000_0000 | 0×4005_FFFF | 384 KB |
|  | Internal ROM 1 | 0×3FF9_0000 | 0×3FF9_FFFF | 64 KB |
|  | Internal SRAM 0 | 0×4007_0000 | 0×4009_FFFF | 192 KB |
|  | Internal SRAM 1 | 0×3FFE_0000 | 0×3FFF_FFFF | 128 KB |
|  |  | 0×400A_0000 | 0×400B_FFFF |  |
|  | Internal SRAM 2 | 0×3FFA_E000 | 0×3FFD_FFFF | 200 KB |
|  | RTC FAST Memory | 0×3FF8_0000 | 0×3FF8_1FFF | 8 KB |
|  |  | 0×400C_0000 | 0×400C_1FFF |  |
|  | RTC SLOW Memory | 0×5000_0000 | 0×5000_1FFF | 8 KB |
| External Memory | External Flash | 0×3F40_0000 | 0×3F7F_FFFF | 4 MB |
|  |  | 0×400C_2000 | 0×40BF_FFFF | 11 MB+248 KB |
|  | External RAM | 0×3F80_0000 | 0×3FBF_FFFF | 4 MB |
| Peripheral | DPort Register | 0×3FF0_0000 | 0×3FF0_0FFF | 4 KB |
|  | AES Accelerator | 0×3FF0_1000 | 0×3FF0_1FFF | 4 KB |
|  | RSA Accelerator | 0×3FF0_2000 | 0×3FF0_2FFF | 4 KB |
|  | SHA Accelerator | 0×3FF0_3000 | 0×3FF0_3FFF | 4 KB |
|  | Secure Boot | 0×3FF0_4000 | 0×3FF0_4FFF | 4 KB |
|  | Cache MMU Table | 0×3FF1_0000 | 0×3FF1_3FFF | 16 KB |
|  | PID Controller | 0×3FF1_F000 | 0×3FF1_FFFF | 4 KB |
|  | UART0 | 0×3FF4_0000 | 0×3FF4_0FFF | 4 KB |
|  | SPI1 | 0×3FF4_2000 | 0×3FF4_2FFF | 4 KB |
|  | SPI0 | 0×3FF4_3000 | 0×3FF4_3FFF | 4 KB |
|  | GPIO | 0×3FF4_4000 | 0×3FF4_4FFF | 4 KB |
|  | RTC | 0×3FF4_8000 | 0×3FF4_8FFF | 4 KB |
|  | IO MUX | 0×3FF4_9000 | 0×3FF4_9FFF | 4 KB |
|  | SDIO Slave | 0×3FF4_B000 | 0×3FF4_BFFF | 4 KB |
|  | UDMA1 | 0×3FF4_C000 | 0×3FF4_CFFF | 4 KB |
|  | I2S0 | 0×3FF4_F000 | 0×3FF4_FFFF | 4 KB |
|  | UART1 | 0×3FF5_0000 | 0×3FF5_0FFF | 4 KB |
|  | I2C0 | 0×3FF5_3000 | 0×3FF5_3FFF | 4 KB |
|  | UDMA0 | 0×3FF5_4000 | 0×3FF5_4FFF | 4 KB |
|  | SDIO Slave | 0×3FF5_5000 | 0×3FF5_5FFF | 4 KB |
|  | RMT | 0×3FF5_6000 | 0×3FF5_6FFF | 4 KB |
|  | PCNT | 0×3FF5_7000 | 0×3FF5_7FFF | 4 KB |
|  | SDIO Slave | 0×3FF5_8000 | 0×3FF5_8FFF | 4 KB |
|  | LED PWM | 0×3FF5_9000 | 0×3FF5_9FFF | 4 KB |
|  | eFuse Controller | 0×3FF5_A000 | 0×3FF5_AFFF | 4 KB |
|  | Flash Encryption | 0×3FF5_B000 | 0×3FF5_BFFF | 4 KB |
|  | PWM0 | 0×3FF5_E000 | 0×3FF5_EFFF | 4 KB |
|  | TIMG0 | 0×3FF5_F000 | 0×3FF5_FFFF | 4 KB |
|  | TIMG1 | 0×3FF6_0000 | 0×3FF6_0FFF | 4 KB |
|  | SPI2 | 0×3FF6_4000 | 0×3FF6_4FFF | 4 KB |
|  | SPI3 | 0×3FF6_5000 | 0×3FF6_5FFF | 4 KB |

| Category | Target | Start Address | End Address | Size |
| --- | --- | --- | --- | --- |
| Peripheral | SYSCON | 0×3FF6_6000 | 0×3FF6_6FFF | 4 KB |
|  | I2C1 | 0×3FF6_7000 | 0×3FF6_7FFF | 4 KB |
|  | SDMMC | 0×3FF6_8000 | 0×3FF6_8FFF | 4 KB |
|  | EMAC | 0×3FF6_9000 | 0×3FF6_AFFF | 8 KB |
|  | TWAI | 0×3FF6_B000 | 0×3FF6_BFFF | 4 KB |
|  | PWM1 | 0×3FF6_C000 | 0×3FF6_CFFF | 4 KB |
|  | I2S1 | 0×3FF6_D000 | 0×3FF6_DFFF | 4 KB |
|  | UART2 | 0×3FF6_E000 | 0×3FF6_EFFF | 4 KB |
|  | PWM2 | 0×3FF6_F000 | 0×3FF6_FFFF | 4 KB |
|  | PWM3 | 0×3FF7_0000 | 0×3FF7_0FFF | 4 KB |
|  | RNG | 0×3FF7_5000 | 0×3FF7_5FFF | 4 KB |

### Cache

ESP32 uses a two-way set-associative cache. Each of the two CPUs has 32 KB of cache featuring a block size of 32 bytes for accessing external storage.

For details, see ESP32 Technical Reference Manual > Chapter System and Memory > Section Cache.

## System Clocks

### CPU Clock

Upon reset, an external crystal clock source is selected as the default CPU clock. The external crystal clock source also connects to a PLL to generate a high-frequency clock (typically 160 MHz).

In addition, ESP32 has an internal 8 MHz oscillator. The application can select the clock source from the external crystal clock source, the PLL clock or the internal 8 MHz oscillator. The selected clock source drives the CPU clock directly, or after division, depending on the application.

### RTC Clock

The RTC clock has five possible sources:

      - External low-speed (32 kHz) crystal clock

      - External crystal clock divided by 4

      - Internal RC oscillator (typically about 150 kHz, and adjustable)

      - Internal 8 MHz oscillator

      - Internal 31.25 kHz clock (derived from the internal 8 MHz oscillator divided by 256)

When the chip is in the normal power mode and needs faster CPU accessing, the application can choose the external high-speed crystal clock divided by 4 or the internal 8 MHz oscillator. When the chip operates in the low-power mode, the application chooses the external low-speed (32 kHz) crystal clock, the internal RC clock or the internal 31.25 kHz clock.

### Audio PLL Clock

The audio clock is generated by the ultra-low-noise fractional-N PLL.

For details, see ESP32 Technical Reference Manual > Chapter Reset and Clock.

## RTC and Low-power Management

### Power Management Unit (PMU)

With the use of advanced power-management technologies, ESP32 can switch between different power modes.

      - Power modes

        - Active mode: The chip radio is powered up. The chip can receive, transmit, or listen.

        - Modem-sleep mode: The CPU is operational and the clock is configurable. The Wi-Fi/Bluetooth baseband and radio are disabled.

        - Light-sleep mode: The CPU is paused. The RTC memory and RTC peripherals, as well as the ULP coprocessor are running. Any wake-up events (MAC, SDIO host, RTC timer, or external interrupts) will wake up the chip.

        - Deep-sleep mode: Only the RTC memory and RTC peripherals are powered up. Wi-Fi and Bluetooth connection data are stored in the RTC memory. The ULP coprocessor is functional.

        - Hibernation mode: The internal 8 MHz oscillator and ULP coprocessor are disabled. The RTC recovery memory is powered down. Only one RTC timer on the slow clock and certain RTC GPIOs are active. The RTC timer or the RTC GPIOs can wake up the chip from the Hibernation mode.

Table 4-2. Power Consumption by Power Modes

| Power mode | Description |  |  | Power Consumption |
| --- | --- | --- | --- | --- |
| Active (RF working) | Wi-Fi Tx packet |  |  | Please refer to Table 5-4 for details. |
|  | Wi-Fi/BT Tx packet |  |  |  |
|  | Wi-Fi/BT Rx and listening |  |  |  |
| Modem-sleep | The CPU is powered up. | 240 MHz * | Dual-core chip(s) | 30 mA ~ 68 mA |
|  |  |  | Single-core chip(s) | N/A |
|  |  | 160 MHz * | Dual-core chip(s) | 27 mA ~ 44 mA |
|  |  |  | Single-core chip(s) | 27 mA ~ 34 mA |
|  |  | Normal speed: 80 MHz | Dual-core chip(s) | 20 mA ~ 31 mA |
|  |  |  | Single-core chip(s) | 20 mA ~ 25 mA |
| Light-sleep | - |  |  | 0.8 mA |
| Deep-sleep | The ULP coprocessor is powered up. |  |  | 150 µA 100 µA @1% duty 10 µA |
|  | ULP sensor-monitored pattern |  |  |  |
|  | RTC timer + RTC memory |  |  |  |
| Hibernation | RTC timer only |  |  | 5 µA |
| Power off | CHIP_PU is set to low level, the chip is powered down. |  |  | 1 µA |

- * Among the ESP32 series of SoCs, ESP32-D0WD-V3, ESP32-D0WDRH2-V3, ESP32-U4WDH, ESP32-D0WD

(NRND), ESP32-D0WDQ6 (NRND), and ESP32-D0WDQ6-V3 (NRND) have a maximum CPU frequency of 240 MHz,

ESP32-S0WD (NRND) has a maximum CPU frequency of 160 MHz.

- When Wi-Fi is enabled, the chip switches between Active and Modem-sleep modes. Therefore, power consumption changes accordingly.

- In Modem-sleep mode, the CPU frequency changes automatically. The frequency depends on the CPU load and the peripherals used.

- During Deep-sleep, when the ULP coprocessor is powered on, peripherals such as GPIO and RTC I2C are able to operate.

- When the system works in the ULP sensor-monitored pattern, the ULP coprocessor works with the ULP sensor periodically and the ADC works with a duty cycle of 1%, so the power consumption is 100 µA.

### Ultra-Low-Power Coprocessor

The ULP coprocessor and RTC memory remain powered on during the Deep-sleep mode. Hence, the developer can store a program for the ULP coprocessor in the RTC slow memory to access the peripheral devices, internal timers and internal sensors during the Deep-sleep mode. This is useful for designing applications where the CPU needs to be woken up by an external event, or a timer, or a combination of the two, while maintaining minimal power consumption.

For details, see ESP32 Technical Reference Manual > Chapter ULP Coprocessor.

## Timers and Watchdogs

### General Purpose Timers

There are four general-purpose timers embedded in the chip. They are all 64-bit generic timers which are based on 16-bit prescalers and 64-bit auto-reload-capable up/down-timers.

The timers feature:

      - A 16-bit clock prescaler, from 2 to 65536

      - A 64-bit timer

      - Configurable up/down timer: incrementing or decrementing

      - Halt and resume of time-base counter

      - Auto-reload at alarming

      - Software-controlled instant reload

      - Level and edge interrupt generation

For details, see ESP32 Technical Reference Manual > Chapter Timer Group.

### Watchdog Timers

The chip has three watchdog timers: one in each of the two timer modules (called the Main Watchdog Timer, or MWDT) and one in the RTC module (called the RTC Watchdog Timer, or RWDT). These watchdog timers are intended to recover from an unforeseen fault causing the application program to abandon its normal sequence. A watchdog timer has four stages. Each stage may trigger one of three or four possible actions upon the expiry of its programmed time period, unless the watchdog is fed or disabled. The actions are:

interrupt, CPU reset, core reset, and system reset. Only the RWDT can trigger the system reset, and is able to reset the entire chip, including the RTC itself. A timeout value can be set for each stage individually.

During flash boot the RWDT and the first MWDT start automatically in order to detect, and recover from, booting problems.

The watchdogs have the following features:

      - Four stages, each of which can be configured or disabled separately

      - A programmable time period for each stage

      - One of three or four possible actions (interrupt, CPU reset, core reset, and system reset) upon the expiry of each stage

      - 32-bit expiry counter

      - Write protection that prevents the RWDT and MWDT configuration from being inadvertently altered

      - SPI flash boot protection

If the boot process from an SPI flash does not complete within a predetermined time period, the watchdog will reboot the entire system.

For details, see ESP32 Technical Reference Manual > Chapter Watchdog Timers.

## Cryptographic Hardware Accelerators

ESP32 is equipped with hardware accelerators of general algorithms, such as AES (FIPS PUB 197), SHA (FIPS PUB 180-4), and RSA. The chip also supports independent arithmetic, such as large-number modular multiplication and large-number multiplication. The maximum operation length for RSA, large-number modular multiplication, and large-number multiplication is 4096 bits.

The hardware accelerators greatly improve operation speed and reduce software complexity. They also support code encryption and dynamic decryption, which ensures that code in the flash will not be hacked.

## Radio and Wi-Fi

The radio module consists of the following blocks:

- 2.4 GHz receiver

- 2.4 GHz transmitter

- Bias and regulators

- Balun and transmit-receive switch

- Clock generator

### 2.4 GHz Receiver

The 2.4 GHz receiver demodulates the 2.4 GHz RF signal to quadrature baseband signals and converts them to the digital domain with two high-resolution, high-speed ADCs. To adapt to varying signal channel conditions, RF filters, Automatic Gain Control (AGC), DC offset cancelation circuits and baseband filters are integrated in the chip.

### 2.4 GHz Transmitter

The 2.4 GHz transmitter modulates the quadrature baseband signals to the 2.4 GHz RF signal, and drives the antenna with a high-powered Complementary Metal Oxide Semiconductor (CMOS) power amplifier. The use of digital calibration further improves the linearity of the power amplifier, enabling state-of-the-art performance in delivering up to +20.5 dBm of power for an 802.11b transmission and +18 dBm for an 802.11n transmission.

Additional calibrations are integrated to cancel any radio imperfections, such as:

      - Carrier leakage

      - I/Q phase matching

      - Baseband nonlinearities

      - RF nonlinearities

      - Antenna matching

These built-in calibration routines reduce the amount of time required for product testing, and render the testing equipment unnecessary.

### Clock Generator

The clock generator produces quadrature clock signals of 2.4 GHz for both the receiver and the transmitter. All components of the clock generator are integrated into the chip, including all inductors, varactors, filters, regulators and dividers.

The clock generator has built-in calibration and self-test circuits. Quadrature clock phases and phase noise are optimized on-chip with patented calibration algorithms which ensure the best performance of the receiver and the transmitter.

### Wi-Fi Radio and Baseband

ESP32 implements a TCP/IP and full 802.11 b/g/n Wi-Fi MAC protocol. It supports the Basic Service Set (BSS) STA and SoftAP operations under the Distributed Control Function (DCF). Power management is handled with minimal host interaction to minimize the active-duty period.

The ESP32 Wi-Fi Radio and Baseband support the following features:

      - 802.11b/g/n

      - 802.11n MCS0-7 in both 20 MHz and 40 MHz bandwidth

      - 802.11n MCS32 (RX)

      - 802.11n 0.4 µs guard-interval

      - up to 150 Mbps of data rate

      - Receiving STBC 2×1

      - Up to 20.5 dBm of transmitting power

      - Adjustable transmitting power

      - Antenna diversity

ESP32 supports antenna diversity with an external RF switch. One or more GPIOs control the RF switch and selects the best antenna to minimize the effects of channel fading.

### Wi-Fi MAC

The ESP32 Wi-Fi MAC applies low-level protocol functions automatically. They are as follows:

      - Four virtual Wi-Fi interfaces

      - Simultaneous Infrastructure BSS Station mode/SoftAP mode/Promiscuous mode

      - RTS protection, CTS protection, Immediate Block ACK

      - Defragmentation

      - TX/RX A-MPDU, RX A-MSDU

      - TXOP

      - WMM

      - CCMP (CBC-MAC, counter mode), TKIP (MIC, RC4), WAPI (SMS4), WEP (RC4) and CRC

      - Automatic beacon monitoring (hardware TSF)

## Bluetooth

The chip integrates a Bluetooth link controller and Bluetooth baseband, which carry out the baseband protocols and other low-level link routines, such as modulation/demodulation, packet processing, bit stream processing, frequency hopping, etc.

### Bluetooth Radio and Baseband

The Bluetooth Radio and Baseband support the following features:

      - Class-1, class-2 and class-3 transmit output powers, and a dynamic control range of up to 21 dB

      - π/4 DQPSK and 8 DPSK modulation

      - High performance in NZIF receiver sensitivity with a minimum sensitivity of -94 dBm

      - Class-1 operation without external PA

      - Internal SRAM allows full-speed data-transfer, mixed voice and data, and full piconet operation

      - Logic for forward error correction, header error control, access code correlation, CRC, demodulation, encryption bit stream generation, whitening and transmit pulse shaping

      - ACL, SCO, eSCO, and AFH

      - A-law, µ-law, and CVSD digital audio CODEC in PCM interface

      - SBC audio CODEC

      - Power management for low-power applications

      - SMP with 128-bit AES

### Bluetooth Interface

      - Provides UART HCI interface, up to 4 Mbps

      - Provides SDIO/SPI HCI interface

      - Provides PCM/I2S audio interface

### Bluetooth Stack

The Bluetooth stack of the chip is compliant with the Bluetooth v4.2 BR/EDR and Bluetooth LE specifications.

### Bluetooth Link Controller

The link controller operates in three major states: standby, connection and sniff. It enables multiple connections, and other operations, such as inquiry, page, and secure simple-pairing, and therefore enables Piconet and Scatternet. Below are the features:

      - Classic Bluetooth

        - Device Discovery (inquiry, and inquiry scan)

        - Connection establishment (page, and page scan)

        - Multi-connections

        - Asynchronous data reception and transmission

        - Synchronous links (SCO/eSCO)

        - Master/Slave Switch

        - Adaptive Frequency Hopping and Channel assessment

        - Broadcast encryption

        - Authentication and encryption

        - Secure Simple-Pairing

        - Multi-point and scatternet management

        - Sniff mode

        - Connectionless Slave Broadcast (transmitter and receiver)

        - Enhanced Power Control

        - Ping

      - Bluetooth Low Energy

        - Advertising

        - Scanning

        - Simultaneous advertising and scanning

        - Multiple connections

        - Asynchronous data reception and transmission

        - Adaptive Frequency Hopping and Channel assessment

        - Connection parameter update

        - Data Length Extension

        - Link Layer Encryption

        - LE Ping

## Digital Peripherals

### General Purpose Input / Output Interface (GPIO)

ESP32 has 34 GPIO pins which can be assigned various functions by programming the appropriate registers. There are several kinds of GPIOs: digital-only, analog-enabled, capacitive-touch-enabled, etc. Analog-enabled GPIOs and Capacitive-touch-enabled GPIOs can be configured as digital GPIOs.

Most of the digital GPIOs can be configured as internal pull-up or pull-down, or set to high impedance. When configured as an input, the input value can be read through the register. The input can also be set to

edge-trigger or level-trigger to generate CPU interrupts. Most of the digital IO pins are bi-directional, non-inverting and tristate, including input and output buffers with tristate control. These pins can be multiplexed with other functions, such as the SDIO, UART, SPI, etc. (More details can be found in the Appendix, Table IO_MUX. ) For low-power operations, the GPIOs can be set to hold their states.

For details, see Section 4.10 Peripheral Pin Configurations, Appendix A –ESP32 Pin Lists and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Serial Peripheral Interface (SPI)

ESP32 integrates four SPI controllers which can be used to communicate with external devices that use the SPI protocol. Controller SPI0 is used as a buffer for accessing external memory. Controller SPI1 can be used as a master. Controllers SPI2 and SPI3 can be configured as either a master or a slave.

SPI1, SPI2, and SPI3 use signal buses prefixed with SPI, HSPI, and VSPI, respectively.

Features of General Purpose SPI (GP-SPI)

      - Programmable data transfer length, in multiples of 1 byte

      - Four-line full-duplex/half-duplex communication and three-line half-duplex communication support

      - Master mode and slave mode

      - Programmable CPOL and CPHA

      - Programmable clock

For details, see ESP32 Technical Reference Manual > Chapter SPI Controller.

Pin Assignment

For SPI, the pins are multiplexed with GPIO6 ~ GPIO11 via the IO MUX. For HSPI, the pins are multiplexed with GPIO2, GPIO4, GPIO12 ~ GPIO15 via the IO MUX. For VSPI, the pins are multiplexed with GPIO5, GPIO18 ~ GPIO19, GPIO21 ~ GPIO23 via the IO MUX.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Universal Asynchronous Receiver Transmitter (UART)

The UART in the ESP32 chip facilitates the transmission and reception of asynchronous serial data between the chip and external UART devices. It consists of two UARTs in the main system, and one low-power LP UART.

Feature List

      - Programmable baud rates up to 5 MBaud

      - RAM shared by TX FIFOs and RX FIFOs

      - Supports input baud rate self-check

      - Support for various lengths of data bits and stop bits

      - Parity bit support

      - Asynchronous communication (RS232 and RS485) and IrDA support

      - Supports DMA to communicate data in high speed

      - Supports UART wake-up

      - Supports both software and hardware flow control

For details, see ESP32 Technical Reference Manual > Chapter UART Controller.

Pin Assignment

The pins for UART can be chosen from any GPIOs via the GPIO Matrix.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### I2C Interface

ESP32 has two I2C bus interfaces which can serve as I2C master or slave, depending on the user’s configuration.

Feature List

      - Two I2C controllers: one in the main system and one in the low-power system

      - Standard mode (100 Kbit/s)

      - Fast mode (400 Kbit/s)

      - Up to 5 MHz, yet constrained by SDA pull-up strength

      - Support for 7-bit and 10-bit addressing, as well as dual address mode

      - Supports continuous data transmission with disabled Serial Clock Line (SCL)

      - Supports programmable digital noise filter

Users can program command registers to control I2C interfaces, so that they have more flexibility. For details, see ESP32 Technical Reference Manual > Chapter I2C Controller.

Pin Assignment

For regular I2C, the pins used can be chosen from any GPIOs via the GPIO Matrix.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### I2S Interface

The I2S Controller in the ESP32 chip provides a flexible communication interface for streaming digital data in multimedia applications, particularly digital audio applications.

Feature List

      - Master mode and slave mode

      - Full-duplex and half-duplex communications

      - A variety of audio standards supported

      - Configurable high-precision output clock

      - Supports PDM signal input and output

      - Configurable data transmit and receive modes

For details, see ESP32 Technical Reference Manual > Chapter I2S Controller.

Pin Assignment

The pins for the I2S Controller can be chosen from any GPIOs via the GPIO Matrix.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Remote Control Peripheral

The Remote Control Peripheral (RMT) controls the transmission and reception of infrared remote control signals.

Feature List

      - Eight channels for sending and receiving infrared remote control signals

      - Independent transmission and reception capabilities for each channel

      - Clock divider counter, state machine, and receiver for each RX channel

      - Supports various infrared protocols

For details, see ESP32 Technical Reference Manual > Chapter Remote Control Peripheral.

Pin Assignment

The pins for the Remote Control Peripheral can be chosen from any GPIOs via the GPIO Matrix. For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Pulse Counter Controller (PCNT)

The pulse counter controller (PCNT) is designed to count input pulses by tracking rising and falling edges of the input pulse signal.

Feature List

      - Eight independent pulse counter units

      - Each pulse counter unit has a 16-bit signed counter register and two channels

      - Counter modes: increment, decrement, or disable

      - Glitch filtering for input pulse signals and control signals

      - Selection between counting on rising or falling edges of the input pulse signal For details, see ESP32 Technical Reference Manual > Chapter Pulse Count Controller.

Pin Assignment

The pins for the Pulse Count Controller can be chosen from any GPIOs via the GPIO Matrix.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### LED PWM Controller

The LED PWM Controller (LEDC) is designed to generate PWM signals for LED control.

Feature List

      - Sixteen independent PWM generators

      - Maximum PWM duty cycle resolution of 20 bits

      - Eight independent timers with 20-bit counters, configurable fractional clock dividers and counter overflow values

      - Adjustable phase of PWM signal output

      - PWM duty cycle dithering

      - Automatic duty cycle fading

For details, see ESP32 Technical Reference Manual > Chapter LED PWM Controller.

Pin Assignment

The pins for the LED PWM Controller can be chosen from any GPIOs via the GPIO Matrix.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Motor Control PWM

The Pulse Width Modulation (PWM) controller can be used for driving digital motors and smart lights. The controller consists of PWM timers, the PWM operator and a dedicated capture sub-module. Each timer provides timing in synchronous or independent form, and each PWM operator generates a waveform for one PWM channel. The dedicated capture sub-module can accurately capture events with external timing.

Feature List

      - Three PWM timers for precise timing and frequency control

        - Every PWM timer has a dedicated 8-bit clock prescaler

        - The 16-bit counter in the PWM timer can work in count-up mode, count-down mode, or count-up-down mode

        - A hardware sync can trigger a reload on the PWM timer with a phase register. It will also trigger the prescaler’restart, so that the timer’s clock can also be synced, with selectable hardware synchronization source

      - Three PWM operators for generating waveform pairs

        - Six PWM outputs to operate in several topologies

        - Configurable dead time on rising and falling edges; each set up independently

        - Modulating of PWM output by high-frequency carrier signals, useful when gate drivers are insulated with a transformer

      - Fault Detection module

        - Programmable fault handling in both cycle-by-cycle mode and one-shot mode

        - A fault condition can force the PWM output to either high or low logic levels

      - Capture module for hardware-based signal processing

        - Speed measurement of rotating machinery

        - Measurement of elapsed time between position sensor pulses

        - Period and duty cycle measurement of pulse train signals

        - Decoding current or voltage amplitude derived from duty-cycle-encoded signals of current/voltage sensors

        - Three individual capture channels, each of which with a 32-bit time-stamp register

        - Selection of edge polarity and prescaling of input capture signals

        - The capture timer can sync with a PWM timer or external signals

For details, see ESP32 Technical Reference Manual > Chapter Motor Control PWM.

Pin Assignment

The pins for the Motor Control PWM can be chosen from any GPIOs via the GPIO Matrix.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### SD/SDIO/MMC Host Controller

An SD/SDIO/MMC host controller is available on ESP32.

Feature List

      - Supports two external cards

      - Supports SD Memory Card standard: version 3.0 and version 3.01)

      - Supports SDIO Version 3.0

      - Supports Consumer Electronics Advanced Transport Architecture (CE-ATA Version 1.1)

      - Supports Multimedia Cards (MMC version 4.41, eMMC version 4.5 and version 4.51)

The controller allows up to 80 MHz clock output in three different data-bus modes: 1-bit, 4-bit, and 8-bit modes. It supports two SD/SDIO/MMC4.41 cards in a 4-bit data-bus mode. It also supports one SD card operating at 1.8 V.

For details, see ESP32 Technical Reference Manual > Chapter SD/MMC Host Controller.

Pin Assignment

The pins for SD/SDIO/MMC Host Controller are multiplexed with GPIO2, GPIO4, GPIO6 ~ GPIO15 via IO MUX.

For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### SDIO/SPI Slave Controller

ESP32 integrates an SD device interface that conforms to the industry-standard SDIO Card Specification Version 2.0, and allows a host controller to access the SoC, using the SDIO bus interface and protocol. ESP32 acts as the slave on the SDIO bus. The host can access the SDIO-interface registers directly and can access shared memory via a DMA engine, thus maximizing performance without engaging the processor cores.

Feature List

The SDIO/SPI slave controller supports the following features:

      - SPI, 1-bit SDIO, and 4-bit SDIO transfer modes over the full clock range from 0 to 50 MHz

      - Configurable sampling and driving clock edge

      - Special registers for direct access by host

      - Interrupts to host for initiating data transfer

      - Automatic loading of SDIO bus data and automatic discarding of padding data

      - Block size of up to 512 bytes

      - Interrupt vectors between the host and the slave, allowing both to interrupt each other

      - Supports DMA for data transfer

For details, see ESP32 Technical Reference Manual > Chapter SDIO Slave Controller.

Pin Assignment

The pins for SDIO/SPI Slave Controller are multiplexed with GPIO2, GPIO4, GPIO6 ~ GPIO15 via IO MUX. For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

    - TWAI® Controller

The Two-wire Automotive Interface (TWAI®) is a multi-master, multi-cast communication protocol designed for automotive applications. The TWAI controller facilitates the communication based on this protocol.

Feature List

      - Compatible with ISO 11898-1 protocol (CAN Specification 2.0)

      - Standard frame format (11-bit ID) and extended frame format (29-bit ID)

      - Bit rates:

        - From 25 Kbit/s to 1 Mbit/s in chip revision v0.0/v1.0/v1.1

        - From 12.5 Kbit/s to 1 Mbit/s in chip revision v3.0/v3.1

      - Multiple modes of operation: Normal, Listen Only, and Self-Test

      - 64-byte receive FIFO

      - Special transmissions: single-shot transmissions and self reception

      - Acceptance filter (single and dual filter modes)

      - Error detection and handling: error counters, configurable error interrupt threshold, error code capture, arbitration lost capture

For details, see ESP32 Technical Reference Manual > Chapter Two-wire Automotive Interface (TWAI).

Pin Assignment

The pins for the Two-wire Automotive Interface can be chosen from any GPIOs via the GPIO Matrix. For more information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Ethernet MAC Interface

An IEEE-802.3-2008-compliant Media Access Controller (MAC) is provided for Ethernet LAN communications. ESP32 requires an external physical interface device (PHY) to connect to the physical LAN bus (twisted-pair, fiber, etc.). The PHY is connected to ESP32 through 17 signals of MII or nine signals of RMII.

Feature List

      - 10 Mbps and 100 Mbps rates

      - Dedicated DMA controller allowing high-speed transfer between the dedicated SRAM and Ethernet MAC

      - Tagged MAC frame (VLAN support)

      - Half-duplex (CSMA/CD) and full-duplex operation

      - MAC control sublayer (control frames)

      - 32-bit CRC generation and removal

      - Several address-filtering modes for physical and multicast address (multicast and group addresses)

      - 32-bit status code for each transmitted or received frame

      - Internal FIFOs to buffer transmit and receive frames. The transmit FIFO and the receive FIFO are both 512 words (32-bit)

      - Hardware PTP (Precision Time Protocol) in accordance with IEEE 1588 2008 (PTP V2)

      - 25 MHz/50 MHz clock output

For details, see ESP32 Technical Reference Manual > Chapter Ethernet Media Access Controller (MAC).

Pin Assignment

For information about the pin assignment of Ethernet MAC Interface, see Section 4.10 Peripheral Pin Configurations and ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

## Analog Peripherals

### Analog-to-Digital Converter (ADC)

ESP32 integrates two 12-bit SAR ADCs and supports measurements on 18 channels (analog-enabled pins). The ULP coprocessor in ESP32 is also designed to measure voltage, while operating in the sleep mode, which enables low-power consumption. The CPU can be woken up by a threshold setting and/or via other

triggers.

Table 4-3 describes the ADC characteristics.

Table 4-3. ADC Characteristics

| Parameter | Description | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| DNL (Differential nonlinearity) | RTC controller; ADC connected to an external 100 nF capacitor; DC signal input; ambient temperature at 25 °C; Wi-Fi&Bluetooth off | –7 | 7 | LSB |
| INL (Integral nonlinearity) |  | –12 | 12 | LSB |
| Sampling rate | RTC controller DIG controller | — | 200 | ksps |
|  |  | — | 2 | Msps |

Notes:

      - When atten = 3 and the measurement result is above 3000 (voltage at approx. 2450 mV), the ADC accuracy will be worse than described in the table above.

      - To get better DNL results, users can take multiple sampling tests with a filter, or calculate the average value.

      - The input voltage range of GPIO pins within VDD3P3_RTC domain should strictly follow the DC characteristics provided in Table 5-3. Otherwise, measurement errors may be introduced, and chip performance may be affected.

By default, there are ±6% differences in measured results between chips. ESP-IDF provides couple of calibration methods for ADC1. Results after calibration using eFuse Vref value are shown in Table 4-4. For higher accuracy, users may apply other calibration methods provided in ESP-IDF, or implement their own.

Table 4-4. ADC Calibration Results

| Parameter | Description | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Total error | Atten = 0, effective measurement range of 100 ∼ 950 mV | –23 | 23 | mV |
|  | Atten = 1, effective measurement range of 100 ∼ 1250 mV | –30 | 30 | mV |
|  | Atten = 2, effective measurement range of 150 ∼ 1750 mV | –40 | 40 | mV |
|  | Atten = 3, effective measurement range of 150 ∼ 2450 mV | –60 | 60 | mV |

For details, see ESP32 Technical Reference Manual > Chapter On-Chip Sensors and Analog Signal Processing.

Pin Assignment

With appropriate settings, the ADCs can be configured to measure voltage on 18 pins maximum. For detailed information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and

ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Digital-to-Analog Converter (DAC)

Two 8-bit DAC channels can be used to convert two digital signals into two analog voltage signal outputs. The design structure is composed of integrated resistor strings and a buffer. This dual DAC supports power supply as input voltage reference. The two DAC channels can also support independent conversions.

For details, see ESP32 Technical Reference Manual > Chapter On-Chip Sensors and Analog Signal Processing.

Pin Assignment

The DAC can be configured by GPIO 25 and GPIO 26. For detailed information about the pin assignment, see Section 4.10 Peripheral Pin Configurations and ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix.

### Touch Sensor

ESP32 has 10 capacitive-sensing GPIOs, which detect variations induced by touching or approaching the GPIOs with a finger or other objects. The low-noise nature of the design and the high sensitivity of the circuit allow relatively small pads to be used. Arrays of pads can also be used, so that a larger area or more points can be detected.

Pin Assignment

The 10 capacitive-sensing GPIOs are listed in Table 4-5.

Table 4-5. Capacitive-Sensing GPIOs Available on ESP32

| Capacitive-Sensing Signal Name | Pin Name |
| --- | --- |
| T0 | GPIO4 |
| T1 | GPIO0 |
| T2 | GPIO2 |
| T3 | MTDO |
| T4 | MTCK |
| T5 | MTDI |
| T6 | MTMS |
| T7 | GPIO27 |
| T8 | 32K_XN |
| T9 | 32K_XP |

For details, see ESP32 Technical Reference Manual > Chapter On-Chip Sensors and Analog Signal Processing.

Note:

ESP32 Touch Sensor has not passed the Conducted Susceptibility (CS) test for now, and thus has limited application scenarios.

## Peripheral Pin Configurations

Table 4-6. Peripheral Pin Configurations

| Interface | Signal | Pin | Function |
| --- | --- | --- | --- |
| ADC | ADC1_CH0 | SENSOR_VP | Two 12-bit SAR ADCs |
|  | ADC1_CH1 | SENSOR_CAPP |  |
|  | ADC1_CH2 | SENSOR_CAPN |  |
|  | ADC1_CH3 | SENSOR_VN |  |
|  | ADC1_CH4 | 32K_XP |  |
|  | ADC1_CH5 | 32K_XN |  |
|  | ADC1_CH6 | VDET_1 |  |
|  | ADC1_CH7 | VDET_2 |  |
|  | ADC2_CH0 | GPIO4 |  |
|  | ADC2_CH1 | GPIO0 |  |
|  | ADC2_CH2 | GPIO2 |  |
|  | ADC2_CH3 | MTDO |  |
|  | ADC2_CH4 | MTCK |  |
|  | ADC2_CH5 | MTDI |  |
|  | ADC2_CH6 | MTMS |  |
|  | ADC2_CH7 | GPIO27 |  |
|  | ADC2_CH8 | GPIO25 |  |
|  | ADC2_CH9 | GPIO26 |  |
| DAC | DAC_1 | GPIO25 | Two 8-bit DACs |
|  | DAC_2 | GPIO26 |  |
| Touch Sensor | TOUCH0 | GPIO4 | Capacitive touch sensors |
|  | TOUCH1 | GPIO0 |  |
|  | TOUCH2 | GPIO2 |  |
|  | TOUCH3 | MTDO |  |
|  | TOUCH4 | MTCK |  |
|  | TOUCH5 | MTDI |  |
|  | TOUCH6 | MTMS |  |
|  | TOUCH7 | GPIO27 |  |
|  | TOUCH8 | 32K_XN |  |
|  | TOUCH9 | 32K_XP |  |
| JTAG | MTDI | MTDI | JTAG for software debugging |
|  | MTCK | MTCK |  |
|  | MTMS | MTMS |  |
|  | MTDO | MTDO |  |

| Interface | Signal | Pin | Function |
| --- | --- | --- | --- |
| SD/SDIO/MMC Host Controller | HS2_CLK | MTMS | Supports SD memory card V3.01 standard |
|  | HS2_CMD | MTDO |  |
|  | HS2_DATA0 | GPIO2 |  |
|  | HS2_DATA1 | GPIO4 |  |
|  | HS2_DATA2 | MTDI |  |
|  | HS2_DATA3 | MTCK |  |
| Motor PWM | PWM0_OUT0~2 | Any GPIO Pins | Three channels of 16-bit timers generate PWM waveforms. Each channel has a pair of output signals, three fault detection signals, three event-capture signals, and three sync signals. |
|  | PWM1_OUT_IN0~2 |  |  |
|  | PWM0_FLT_IN0~2 |  |  |
|  | PWM1_FLT_IN0~2 |  |  |
|  | PWM0_CAP_IN0~2 |  |  |
|  | PWM1_CAP_IN0~2 |  |  |
|  | PWM0_SYNC_IN0~2 |  |  |
|  | PWM1_SYNC_IN0~2 |  |  |
| SDIO/SPI Slave Controller | SD_CLK | MTMS | SDIO interface that conforms to the industry standard SDIO 2.0 card specification |
|  | SD_CMD | MTDO |  |
|  | SD_DATA0 | GPIO2 |  |
|  | SD_DATA1 | GPIO4 |  |
|  | SD_DATA2 | MTDI |  |
|  | SD_DATA3 | MTCK |  |
| UART | U0RXD_in | Any GPIO Pins | Three UART devices with hardware flow-control and DMA |
|  | U0CTS_in |  |  |
|  | U0DSR_in |  |  |
|  | U0TXD_out |  |  |
|  | U0RTS_out |  |  |
|  | U0DTR_out |  |  |
|  | U1RXD_in |  |  |
|  | U1CTS_in |  |  |
|  | U1TXD_out |  |  |
|  | U1RTS_out |  |  |
|  | U2RXD_in |  |  |
|  | U2CTS_in |  |  |
|  | U2TXD_out |  |  |
|  | U2RTS_out |  |  |
| I2C | I2CEXT0_SCL_in | Any GPIO Pins | Two I2C devices in slave or master mode |
|  | I2CEXT0_SDA_in |  |  |
|  | I2CEXT1_SCL_in |  |  |
|  | I2CEXT1_SDA_in |  |  |
|  | I2CEXT0_SCL_out |  |  |
|  | I2CEXT0_SDA_out |  |  |
|  | I2CEXT1_SCL_out |  |  |
|  | I2CEXT1_SDA_out |  |  |

| Interface | Signal | Pin | Function |
| --- | --- | --- | --- |
| LED PWM | ledc_hs_sig_out0~7 | Any GPIO Pins | 16 independent channels @80 MHz clock/RTC CLK. Duty accuracy: 16 bits. |
|  | ledc_ls_sig_out0~7 |  |  |
| I2S | I2S0I_DATA_in0~15 | Any GPIO Pins | Stereo input and output from/to the audio codec; parallel LCD data output; parallel camera data input. Note: I2S0_CLK and I2S1_CLK can only be mapped to GPIO0, U0RXD (GPIO3), or U0TXD (GPIO1) via IO MUX by selecting GPIO functions CLK_OUT1, CLK_OUT2, and CLK_OUT3. For more information, see ESP32 Technical Reference Manual > Chapter IO_MUX and GPIO Matrix > Table IO MUX Pad Summary. |
|  | I2S0O_BCK_in |  |  |
|  | I2S0O_WS_in |  |  |
|  | I2S0I_BCK_in |  |  |
|  | I2S0I_WS_in |  |  |
|  | I2S0I_H_SYNC |  |  |
|  | I2S0I_V_SYNC |  |  |
|  | I2S0I_H_ENABLE |  |  |
|  | I2S0O_BCK_out |  |  |
|  | I2S0O_WS_out |  |  |
|  | I2S0I_BCK_out |  |  |
|  | I2S0I_WS_out |  |  |
|  | I2S0O_DATA_out0~23 |  |  |
|  | I2S1I_DATA_in0~15 |  |  |
|  | I2S1O_BCK_in |  |  |
|  | I2S1O_WS_in |  |  |
|  | I2S1I_BCK_in |  |  |
|  | I2S1I_WS_in |  |  |
|  | I2S1I_H_SYNC |  |  |
|  | I2S1I_V_SYNC |  |  |
|  | I2S1I_H_ENABLE |  |  |
|  | I2S1O_BCK_out |  |  |
|  | I2S1O_WS_out |  |  |
|  | I2S1I_BCK_out |  |  |
|  | I2S1I_WS_out |  |  |
|  | I2S1O_DATA_out0~23 |  |  |
|  | I2S0_CLK | GPIO0, U0RXD, or U0TXD |  |
|  | I2S1_CLK |  |  |
| RMT | RMT_SIG_IN0~7 | Any GPIO Pins | Eight channels for an IR transmitter and receiver of various waveforms |
|  | RMT_SIG_OUT0~7 |  |  |
| General Purpose SPI | HSPIQ_in/_out | Any GPIO Pins | Standard SPI consists of clock, chip-select, MOSI and MISO. These SPIs can be connected to LCD and other external devices. They support the following features: - Both master and slave modes; - Four sub-modes of the SPI transfer format; - Configurable SPI frequency; - Up to 64 bytes of FIFO and DMA. |
|  | HSPID_in/_out |  |  |
|  | HSPICLK_in/_out |  |  |
|  | HSPI_CS0_in/_out |  |  |
|  | HSPI_CS1_out |  |  |
|  | HSPI_CS2_out |  |  |
|  | VSPIQ_in/_out |  |  |
|  | VSPID_in/_out |  |  |
|  | VSPICLK_in/_out |  |  |
|  | VSPI_CS0_in/_out |  |  |
|  | VSPI_CS1_out |  |  |
|  | VSPI_CS2_out |  |  |

| Interface | Signal | Pin | Function |
| --- | --- | --- | --- |
| Parallel QSPI | SPIHD | SD_DATA_2 | Supports Standard SPI, Dual SPI, and Quad SPI that can be connected to the external flash and SRAM |
|  | SPIWP | SD_DATA_3 |  |
|  | SPICS0 | SD_CMD |  |
|  | SPICLK | SD_CLK |  |
|  | SPIQ | SD_DATA_0 |  |
|  | SPID | SD_DATA_1 |  |
|  | HSPICLK | MTMS |  |
|  | HSPICS0 | MTDO |  |
|  | HSPIQ | MTDI |  |
|  | HSPID | MTCK |  |
|  | HSPIHD | GPIO4 |  |
|  | HSPIWP | GPIO2 |  |
|  | VSPICLK | GPIO18 |  |
|  | VSPICS0 | GPIO5 |  |
|  | VSPIQ | GPIO19 |  |
|  | VSPID | GPIO23 |  |
|  | VSPIHD | GPIO21 |  |
|  | VSPIWP | GPIO22 |  |
| EMAC | EMAC_TX_CLK | GPIO0 | Ethernet MAC with MII/RMII interface |
|  | EMAC_RX_CLK | GPIO5 |  |
|  | EMAC_TX_EN | GPIO21 |  |
|  | EMAC_TXD0 | GPIO19 |  |
|  | EMAC_TXD1 | GPIO22 |  |
|  | EMAC_TXD2 | MTMS |  |
|  | EMAC_TXD3 | MTDI |  |
|  | EMAC_RX_ER | MTCK |  |
|  | EMAC_RX_DV | GPIO27 |  |
|  | EMAC_RXD0 | GPIO25 |  |
|  | EMAC_RXD1 | GPIO26 |  |
|  | EMAC_RXD2 | U0TXD |  |
|  | EMAC_RXD3 | MTDO |  |
|  | EMAC_CLK_OUT | GPIO16 |  |
|  | EMAC_CLK_OUT_180 | GPIO17 |  |
|  | EMAC_TX_ER | GPIO4 |  |
|  | EMAC_MDC_out | Any GPIO Pins |  |
|  | EMAC_MDI_in | Any GPIO Pins |  |
|  | EMAC_MDO_out | Any GPIO Pins |  |
|  | EMAC_CRS_out | Any GPIO Pins |  |
|  | EMAC_COL_out | Any GPIO Pins |  |

| Interface | Signal | Pin | Function |
| --- | --- | --- | --- |
| Pulse Counter | pcnt_sig_ch0_in0 | Any GPIO Pins | Operating in seven different modes, the pulse counter captures pulse and counts pulse edges. |
|  | pcnt_sig_ch1_in0 |  |  |
|  | pcnt_ctrl_ch0_in0 |  |  |
|  | pcnt_ctrl_ch1_in0 |  |  |
|  | pcnt_sig_ch0_in1 |  |  |
|  | pcnt_sig_ch1_in1 |  |  |
|  | pcnt_ctrl_ch0_in1 |  |  |
|  | pcnt_ctrl_ch1_in1 |  |  |
|  | pcnt_sig_ch0_in2 |  |  |
|  | pcnt_sig_ch1_in2 |  |  |
|  | pcnt_ctrl_ch0_in2 |  |  |
|  | pcnt_ctrl_ch1_in2 |  |  |
|  | pcnt_sig_ch0_in3 |  |  |
|  | pcnt_sig_ch1_in3 |  |  |
|  | pcnt_ctrl_ch0_in3 |  |  |
|  | pcnt_ctrl_ch1_in3 |  |  |
|  | pcnt_sig_ch0_in4 |  |  |
|  | pcnt_sig_ch1_in4 |  |  |
|  | pcnt_ctrl_ch0_in4 |  |  |
|  | pcnt_ctrl_ch1_in4 |  |  |
|  | pcnt_sig_ch0_in5 |  |  |
|  | pcnt_sig_ch1_in5 |  |  |
|  | pcnt_ctrl_ch0_in5 |  |  |
|  | pcnt_ctrl_ch1_in5 |  |  |
|  | pcnt_sig_ch0_in6 |  |  |
|  | pcnt_sig_ch1_in6 |  |  |
|  | pcnt_ctrl_ch0_in6 |  |  |
|  | pcnt_ctrl_ch1_in6 |  |  |
|  | pcnt_sig_ch0_in7 |  |  |
|  | pcnt_sig_ch1_in7 |  |  |
|  | pcnt_ctrl_ch0_in7 |  |  |
|  | pcnt_ctrl_ch1_in7 |  |  |
| TWAI | twai_rx | Any GPIO Pins | Compatible with ISO 11898-1 protocol (CAN Specification 2.0) |
|  | twai_tx |  |  |
|  | twai_bus_off_on |  |  |
|  | twai_clkout |  |  |

# Electrical Characteristics

## Absolute Maximum Ratings

Stresses above those listed in Table 5-1 Absolute Maximum Ratings may cause permanent damage to the device. These are stress ratings only and normal operation of the device at these or any other conditions beyond those indicated in Section 5.2 Recommended Power Supply Characteristics is not implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability.

Table 5-1. Absolute Maximum Ratings

| Parameter | Description | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| VDDA, VDD3P3, VDD3P3_RTC, VDD3P3_CPU, VDD_SDIO | Allowed input voltage | –0.3 | 3.6 | V |
| Ioutput1 | Cumulative IO output current | — | 1200 | mA |
| TSTORE | Storage temperature | –40 | 150 | °C |

1 The product proved to be fully functional after all its IO pins were pulled high while being connected to ground for 24 consecutive hours at ambient temperature of 25 °C.

## Recommended Power Supply Characteristics

Table 5-2. Recommended Power Supply Characteristics

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| VDDA, VDD3P3_RTC, VDD3P3, VDD_SDIO (3.3 V mode) note 1 | Voltage applied to power supply pins per power domain | 2.3/3.0 note 2 | 3.3 | 3.6 | V |
| VDD3P3_CPU | Voltage applied to power supply pin | 1.8 | 3.3 | 3.6 | V |
| IV DD | Current delivered by external power supply | 0.5 | — | — | A |
| T note 3 | Operating temperature | –40 | — | 125 | °C |

- • VDD_SDIO works as the power supply for the related IO, and also for an external device. Please refer to the Appendix IO_MUX of this datasheet for more details.

  - VDD_SDIO can be sourced internally by the ESP32 from the VDD3P3_RTC power domain:

    - When VDD_SDIO operates at 3.3 V, it is driven directly by VDD3P3_RTC through a 6 Ω resistor, therefore, there will be some voltage drop from VDD3P3_RTC.

    - When VDD_SDIO operates at 1.8 V, it can be generated from ESP32’s internal LDO. The maximum current this LDO can offer is 40 mA, and the output voltage range is 1.65 V ~ 2.0 V.

  - VDD_SDIO can also be driven by an external power supply.

  - Please refer to Section 2.5.2 Power Scheme, for more information.

- • Chips with a 3.3 V flash or PSRAM in-package: this minimum voltage is 3.0 V;

  - Chips with no flash or PSRAM in-package: this minimum voltage is 2.3 V;

  - For more information, see Section 1 ESP32 Series Comparison.

- The operating temperature of ESP32-U4WDH and ESP32-D0WDRH2-V3 ranges from –40 °C to 85 °C, due to the

in-package flash or PSRAM. For other chips that have no in-package flash or PSRAM, their operating temperature is

–40 °C ~ 125 °C.

  - DC Characteristics (3.3 V, 25 °C)

Table 5-3. DC Characteristics (3.3 V, 25 °C)

| Parameter | Description |  | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| CIN | Pin capacitance |  | — | 2 | — | pF |
| VIH | High-level input voltage |  | 0.75 × VDD 1 | — | VDD 1 + 0.3 | V |
| VIL | Low-level input voltage |  | –0.3 | — | 0.25 × VDD 1 | V |
| IIH | High-level input current |  | — | — | 50 | nA |
| IIL | Low-level input current |  | — | — | 50 | nA |
| VOH | High-level output voltage |  | 0.8 × VDD 1 | — | — | V |
| VOL | Low-level output voltage |  | — | — | 0.1 × VDD 1 | V |
| IOH | High-level source current (VDD 1 = 3.3 V, VOH >= 2.64 V, output drive strength set to the maximum) | VDD3P3_CPU power domain 1, 2 | — | 40 | — | mA |
|  |  | VDD3P3_RTC power domain 1, 2 | — | 40 | — | mA |
|  |  | VDD_SDIO power domain 1, 3 | — | 20 | — | mA |
| IOL | Low-level sink current (VDD 1 = 3.3 V, VOL = 0.495 V, output drive strength set to the maximum) |  | — | 28 | — | mA |
| RPU | Resistance of internal pull-up resistor |  | — | 45 | — | kΩ |
| RPD | Resistance of internal pull-down resistor |  | — | 45 | — | kΩ |
| VIH_nRST | Chip reset release voltage (CHIP_PU voltage is within the specified range) |  | 0.75 × VDD 1 | — | VDD 1 + 0.3 | V |
| VIL_nRST | Low-level input voltage of CHIP_PU to shut down the chip |  | — | — | 0.6 | V |

- Please see Table IO_MUX for IO’s power domain. VDD is the I/O voltage for a particular power domain of pins.

- For VDD3P3_CPU and VDD3P3_RTC power domain, per-pin current sourced in the same domain is gradually reduced from around 40 mA to around 29 mA, VOH >=2.64 V, as the number of current-source pins increases.

- For VDD_SDIO power domain, per-pin current sourced in the same domain is gradually reduced from around 30 mA to around 10 mA, VOH >=2.64 V, as the number of current-source pins increases.

## RF Current Consumption in Active Mode

The current consumption measurements are taken with a 3.3 V supply at 25 °C of ambient temperature at the RF port. All transmitters’ measurements are based on a 50% duty cycle.

Table 5-4. Current Consumption Depending on RF Modes

| Work Mode | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- |
| Transmit 802.11b, DSSS 1 Mbps, POUT = +19.5 dBm | — | 240 | — | mA |
| Transmit 802.11g, OFDM 54 Mbps, POUT = +16 dBm | — | 190 | — | mA |
| Transmit 802.11n, OFDM MCS7, POUT = +14 dBm | — | 180 | — | mA |
| Receive 802.11b/g/n | — | 95 ~ 100 | — | mA |

| Work Mode | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- |
| Transmit BT/BLE, POUT = 0 dBm | — | 130 | — | mA |
| Receive BT/BLE | — | 95 ~ 100 | — | mA |

## Reliability

Table 5-5. Reliability Qualifications

| Test Item | Test Conditions | Test Standard |
| --- | --- | --- |
| HTOL (High Temperature Operating Life) | 125 °C, 1000 hours | JESD22-A108 |
| ESD (Electro-Static Discharge Sensitivity) | HBM (Human Body Mode) 1 ± 2000 V | JS-001 |
|  | CDM (Charge Device Mode) 2 ± 500 V | JS-002 |
| Latch up | Current trigger ± 200 mA | JESD78 |
|  | Voltage trigger 1.5 × VDDmax |  |
| Preconditioning | Bake 24 hours @125 °C Moisture soak (level 3: 192 hours @30 °C, 60% RH) IR reflow solder: 260 + 0 °C, 20 seconds, three times | J-STD-020, JESD47, JESD22-A113 |
| TCT (Temperature Cycling Test) | –65 °C / 150 °C, 500 cycles | JESD22-A104 |
| Autoclave Test | 121 °C, 100% RH, 96 hours | JESD22-A102 |
| uHAST (Highly Accel- erated Stress Test, unbiased) | 130 °C, 85% RH, 96 hours | JESD22-A118 |
| HTSL (High Temperature Storage Life) | 150 °C, 1000 hours | JESD22-A103 |

- JEDEC document JEP155 states that 500 V HBM allows safe manufacturing with a standard ESD control process.

- JEDEC document JEP157 states that 250 V CDM allows safe manufacturing with a standard ESD control process.

## Wi-Fi Radio

Table 5-6. Wi-Fi Radio Characteristics

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Operating frequency range note1 | — | 2412 | — | 2484 | MHz |
| Output impedance note2 | - | - | note 2 | — | Ω |
| TX power note3 | 11n, MCS7 | 12 | 13 | 14 | dBm |
|  | 11b mode | 18.5 | 19.5 | 20.5 | dBm |
| Sensitivity | 11b, 1 Mbps | — | –98 | — | dBm |
|  | 11b, 11 Mbps | — | –88 | — | dBm |
|  | 11g, 6 Mbps | — | –93 | — | dBm |
|  | 11g, 54 Mbps | — | –75 | — | dBm |

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
|  | 11n, HT20, MCS0 | — | –93 | — | dBm |
|  | 11n, HT20, MCS7 | — | –73 | — | dBm |
|  | 11n, HT40, MCS0 | — | –90 | — | dBm |
|  | 11n, HT40, MCS7 | — | –70 | — | dBm |
| Adjacent channel rejection | 11g, 6 Mbps | — | 27 | — | dB |
|  | 11g, 54 Mbps | — | 13 | — | dB |
|  | 11n, HT20, MCS0 | — | 27 | — | dB |
|  | 11n, HT20, MCS7 | — | 12 | — | dB |

- Device should operate in the frequency range allocated by regional regulatory authorities. Target operating frequency range is configurable by software.

- The typical value of the Wi-Fi radio output impedance is different between chips in different QFN packages. For chips in a QFN 6×6 package, the value is 30+j10 Ω. For chips in a QFN 5×5 package, the value is 35+j10 Ω.

- Target TX power is configurable based on device or certification requirements.

## Bluetooth Radio

### Receiver –Basic Data Rate

Table 5-7. Receiver Characteristics –Basic Data Rate

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Sensitivity @0.1% BER | — | –90 | –89 | –88 | dBm |
| Maximum received signal @0.1% BER | — | 0 | — | — | dBm |
| Co-channel C/I | — | — | +7 | — | dB |
| Adjacent channel selectivity C/I | F = F0 + 1 MHz | — | — | –6 | dB |
|  | F = F0 –1 MHz | — | — | –6 | dB |
|  | F = F0 + 2 MHz | — | — | –25 | dB |
|  | F = F0 –2 MHz | — | — | –33 | dB |
|  | F = F0 + 3 MHz | — | — | –25 | dB |
|  | F = F0 –3 MHz | — | — | –45 | dB |
| Out-of-band blocking performance | 30 MHz ~ 2000 MHz | –10 | — | — | dBm |
|  | 2000 MHz ~ 2400 MHz | –27 | — | — | dBm |
|  | 2500 MHz ~ 3000 MHz | –27 | — | — | dBm |
|  | 3000 MHz ~ 12.5 GHz | –10 | — | — | dBm |
| Intermodulation | — | –36 | — | — | dBm |

### Transmitter –Basic Data Rate

Table 5-8. Transmitter Characteristics –Basic Data Rate

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| RF transmit power note1 | — | — | 0 | — | dBm |
| Gain control step | — | — | 3 | — | dB |

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| RF power control range | — | –12 | — | +9 | dBm |
| +20 dB bandwidth | — | — | 0.9 | — | MHz |
| Adjacent channel transmit power | F = F0 ± 2 MHz | — | –47 | — | dBm |
|  | F = F0 ± 3 MHz | — | –55 | — | dBm |
|  | F = F0 ± > 3 MHz | — | –60 | — | dBm |
| ∆ f 1avg | — | — | — | 155 | kHz |
| ∆ f 2max | — | 133.7 | — | — | kHz |
| ∆ f 2avg/∆ f 1avg | — | — | 0.92 | — | — |
| ICFT | — | — | –7 | — | kHz |
| Drift rate | — | — | 0.7 | — | kHz/50 µs |
| Drift (DH1) | — | — | 6 | — | kHz |
| Drift (DH5) | — | — | 6 | — | kHz |

1. There are in total eight power levels from level 0 to level 7, with transmit power ranging from –12 dBm to 9 dBm. When the power level rises by 1, the transmit power increases by 3 dB. Power level 4 is used by default and the corresponding transmit power is 0 dBm.

### Receiver –Enhanced Data Rate

Table 5-9. Receiver Characteristics –Enhanced Data Rate

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| π/4 DQPSK |  |  |  |  |  |
| Sensitivity @0.01% BER | — | –90 | –89 | –88 | dBm |
| Maximum received signal @0.01% BER | — | — | 0 | — | dBm |
| Co-channel C/I | — | — | 11 | — | dB |
| Adjacent channel selectivity C/I | F = F0 + 1 MHz | — | –7 | — | dB |
|  | F = F0 –1 MHz | — | –7 | — | dB |
|  | F = F0 + 2 MHz | — | –25 | — | dB |
|  | F = F0 –2 MHz | — | –35 | — | dB |
|  | F = F0 + 3 MHz | — | –25 | — | dB |
|  | F = F0 –3 MHz | — | –45 | — | dB |
| 8DPSK |  |  |  |  |  |
| Sensitivity @0.01% BER | — | –84 | –83 | –82 | dBm |
| Maximum received signal @0.01% BER | — | — | –5 | — | dBm |
| C/I c-channel | — | — | 18 | — | dB |
| Adjacent channel selectivity C/I | F = F0 + 1 MHz | — | 2 | — | dB |
|  | F = F0 –1 MHz | — | 2 | — | dB |
|  | F = F0 + 2 MHz | — | –25 | — | dB |
|  | F = F0 –2 MHz | — | –25 | — | dB |
|  | F = F0 + 3 MHz | — | –25 | — | dB |
|  | F = F0 –3 MHz | — | –38 | — | dB |

### Transmitter –Enhanced Data Rate

Table 5-10. Transmitter Characteristics –Enhanced Data Rate

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| RF transmit power (see note under Table 5-10) | — | — | 0 | — | dBm |
| Gain control step | — | — | 3 | — | dB |
| RF power control range | — | –12 | — | +9 | dBm |
| π/4 DQPSK max w0 | — | — | –0.72 | — | kHz |
| π/4 DQPSK max wi | — | — | –6 | — | kHz |
| π/4 DQPSK max ¦wi + w0¦ | — | — | –7.42 | — | kHz |
| 8DPSK max w0 | — | — | 0.7 | — | kHz |
| 8DPSK max wi | — | — | –9.6 | — | kHz |
| 8DPSK max ¦wi + w0¦ | — | — | –10 | — | kHz |
| π/4 DQPSK modulation accuracy | RMS DEVM | — | 4.28 | — | % |
|  | 99% DEVM | — | 100 | — | % |
|  | Peak DEVM | — | 13.3 | — | % |
| 8 DPSK modulation accuracy | RMS DEVM | — | 5.8 | — | % |
|  | 99% DEVM | — | 100 | — | % |
|  | Peak DEVM | — | 14 | — | % |
| In-band spurious emissions | F = F0 ± 1 MHz | — | –46 | — | dBm |
|  | F = F0 ± 2 MHz | — | –40 | — | dBm |
|  | F = F0 ± 3 MHz | — | –46 | — | dBm |
|  | F = F0 +/–> 3 MHz | — | — | –53 | dBm |
| EDR differential phase coding | — | — | 100 | — | % |

## Bluetooth LE Radio

### Receiver

Table 5-11. Receiver Characteristics –Bluetooth LE

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Sensitivity @30.8% PER | — | –94 | –93 | –92 | dBm |
| Maximum received signal @30.8% PER | — | 0 | — | — | dBm |
| Co-channel C/I | — | — | +10 | — | dB |
| Adjacent channel selectivity C/I | F = F0 + 1 MHz | — | –5 | — | dB |
|  | F = F0 –1 MHz | — | –5 | — | dB |
|  | F = F0 + 2 MHz | — | –25 | — | dB |
|  | F = F0 –2 MHz | — | –35 | — | dB |
|  | F = F0 + 3 MHz | — | –25 | — | dB |
|  | F = F0 –3 MHz | — | –45 | — | dB |
| Out-of-band blocking performance | 30 MHz ~ 2000 MHz | –10 | — | — | dBm |
|  | 2000 MHz ~ 2400 MHz | –27 | — | — | dBm |
|  | 2500 MHz ~ 3000 MHz | –27 | — | — | dBm |

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
|  | 3000 MHz ~ 12.5 GHz | –10 | — | — | dBm |
| Intermodulation | — | –36 | — | — | dBm |

### Transmitter

Table 5-12. Transmitter Characteristics –Bluetooth LE

| Parameter | Description | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| RF transmit power (see note under Table 5-8) | — | — | 0 | — | dBm |
| Gain control step | — | — | 3 | — | dB |
| RF power control range | — | –12 | — | +9 | dBm |
| Adjacent channel transmit power | F = F0 ± 2 MHz | — | –52 | — | dBm |
|  | F = F0 ± 3 MHz | — | –58 | — | dBm |
|  | F = F0 ± > 3 MHz | — | –60 | — | dBm |
| ∆ f 1avg | — | — | — | 265 | kHz |
| ∆ f 2max | — | 247 | — | — | kHz |
| ∆ f 2avg/∆ f 1avg | — | — | 0.92 | — | — |
| ICFT | — | — | –10 | — | kHz |
| Drift rate | — | — | 0.7 | — | kHz/50 µs |
| Drift | — | — | 2 | — | kHz |

in 1

Pn 2

# Packaging

- For information about tape, reel, and chip marking, please refer to ESP32 Chip Packaging Information.

- The pins of the chip are numbered in anti-clockwise order starting from Pin 1 in the top view. For pin numbers and pin names, see also pin layout figures in Section 2.1 Pin Layout.

Pin 1

Pin 2

Pin 3

Pin 1

Pin 2

Pin 3

Figure 6-1. QFN48 (6×6 mm) Package

1//lccclCI

0 a.a.a . C

48X

e

L  0.300 0.350 0.400

Tol. of Form&Position

0.350 BSC

0 a.a.a. C

BOTTOM VIEw

aaa

bl

bbb

CCC

ddd eee fff

0.10

0.10

0.10

0.05

0.08

0.10

Notes

SIDE VIEw

2ı DIMENSIONING AND T□LERANCING PER JEDEC M□-220ı

48L SLP

(5x5r1r1)

D

PIN #1 DDT

BY MARKING

0.050

D2

PIN #1 ID

C01350

L

Dimensional Ref.

REF. Min.	Norn. Max. A  0.800 0.850 0.900

A1 0.000

--

E2 3.650 3.700 3.750

b  0.130 0.180 0.230

E

L

e_f

L

+

A3	0.203 Ref.

E2

D  4.950 5.000 5.050

E 4.950 5.000 5.050

02 3.650 3.700 3.750

TOP VIEw

1. All DIMENSIONS ARE IN MILLIMETERS,

b1 0.070 0.120 0.170

Figure 6-2. QFN48 (5×5 mm) Package

Related Documentation and Resources

# Related Documentation and Resources

## Related Documentation

- ESP32 Technical Reference Manual – Detailed information on how to use the ESP32 memory and peripherals.

- ESP32 Hardware Design Guidelines – Guidelines on how to integrate the ESP32 into your hardware product.

- ESP32 ECO and Workarounds for Bugs – Correction of ESP32 design errors.

- ESP32 Series SoC Errata – Descriptions of known errors in ESP32 series of SoCs.

- Certificates

https://espressif.com/en/support/documents/certificates

- ESP32 Product/Process Change Notifications (PCN)

https://espressif.com/en/support/documents/pcns

- ESP32 Advisories – Information on security, bugs, compatibility, component reliability.

https://espressif.com/en/support/documents/advisories

- Documentation Updates and Update Notification Subscription

https://espressif.com/en/support/download/documents

## Developer Zone

- ESP-IDF Programming Guide for ESP32 – Extensive documentation for the ESP-IDF development framework.

- ESP-IDF and other development frameworks on GitHub. https://github.com/espressif

- ESP32 BBS Forum – Engineer-to-Engineer (E2E) Community for Espressif products where you can post questions, share knowledge, explore ideas, and help solve problems with fellow engineers.

https://esp32.com/

- The ESP Journal – Best Practices, Articles, and Notes from Espressif folks.

https://blog.espressif.com/

- See the tabs SDKs and Demos, Apps, Tools, AT Firmware. https://espressif.com/en/support/download/sdks-demos

## Products

- ESP32 Series SoCs – Browse through all ESP32 SoCs.

https://espressif.com/en/products/socs?id=ESP32

- ESP32 Series Modules – Browse through all ESP32-based modules.

https://espressif.com/en/products/modules?id=ESP32

- ESP32 Series DevKits – Browse through all ESP32-based devkits.

https://espressif.com/en/products/devkits?id=ESP32

- ESP Product Selector – Find an Espressif hardware product suitable for your needs by comparing or applying filters.

https://products.espressif.com/#/product-selector?language=en

## Contact Us

- See the tabs Sales Questions, Technical Enquiries, Circuit Schematic & PCB Design Review, Get Samples

(Online stores), Become Our Supplier, Comments & Suggestions. https://espressif.com/en/contact-us/sales-questions

# Appendix A –ESP32 Pin Lists

## Notes on ESP32 Pin Lists

Table 6-1. Notes on ESP32 Pin Lists

| No. | Description |
| --- | --- |
| 1 | In Table IO_MUX, the boxes highlighted in yellow indicate the GPIO pins that are input-only. Please see the following note for further details. |
| 2 | GPIO pins 34-39 are input-only. These pins do not feature an output driver or internal pull-up/pull-down circuitry. The pin names are: SENSOR_VP (GPIO36), SENSOR_CAPP (GPIO37), SENSOR_CAPN (GPIO38), SENSOR_VN (GPIO39), VDET_1 (GPIO34), VDET_2 (GPIO35). |
| 3 | The pins are grouped into four power domains: VDDA (analog power supply), VDD3P3_RTC (RTC power supply), VDD3P3_CPU (power supply of digital IOs and CPU cores), VDD_SDIO (power supply of SDIO IOs). VDD_SDIO is the output of the internal SDIO-LDO. The voltage of SDIO-LDO can be configured at 1.8 V or be the same as that of VDD3P3_RTC. The strapping pin and eFuse bits determine the default voltage of the SDIO-LDO. Software can change the voltage of the SDIO-LDO by configuring register bits. For details, please see the column “Power Domain”in Table IO_MUX. |
| 4 | The functional pins in the VDD3P3_RTC domain are those with analog functions, including the 32 kHz crystal oscillator, ADC, DAC, and the capacitive touch sensor. Please see columns “Analog Function 0 ~ 2” in Table IO_MUX. |
| 5 | These VDD3P3_RTC pins support the RTC function, and can work during Deep-sleep. For example, an RTC-GPIO can be used for waking up the chip from Deep-sleep. |
| 6 | The GPIO pins support up to six digital functions, as shown in columns “Function 0 ~ 5” In Table IO_MUX. The function selection registers will be set as “N”, where N is the function number. Below are some definitions: - SD_* is for signals of the SDIO slave. - HS1_* is for Port 1 signals of the SDIO host. - HS2_* is for Port 2 signals of the SDIO host. - MT* is for signals of the JTAG. - U0* is for signals of the UART0 module. - U1* is for signals of the UART1 module. - U2* is for signals of the UART2 module. - SPI* is for signals of the SPI01 module. - HSPI* is for signals of the SPI2 module. - VSPI* is for signals of the SPI3 module. |

| No. | Description |
| --- | --- |
| 7 | Each column about digital “Function” is accompanied by a column about “Type”. Please see the following explanations for the meanings of“type” with respect to each“function” they are associated with. For each “Function-N”, “type” signifies: - I: input only. If a function other than “Function-N” is assigned, the input signal of “Function-N” is still from this pin. - I1: input only. If a function other than “Function-N” is assigned, the input signal of “Function-N” is always “1”. - I0: input only. If a function other than “Function-N” is assigned, the input signal of “Function-N” is always “0”. - O: output only. - T: high-impedance. - I/O/T: combinations of input, output, and high-impedance according to the function signal. - I1/O/T: combinations of input, output, and high-impedance, according to the function signal. If a function is not selected, the input signal of the function is “1”. For example, pin 30 can function as HS1_CMD or SD_CMD, where HS1_CMD is of an“I1/O/T” type. If pin 30 is selected as HS1_CMD, this pin’s input and output are controlled by the SDIO host. If pin 30 is not selected as HS1_CMD, the input signal of the SDIO host is always “1”. |
| 8 | Each digital output pin is associated with its configurable drive strength. Column “Drive Strength” in Table IO_MUX lists the default values. The drive strength of the digital output pins can be configured into one of the following four options: - 0: ~5 mA - 1: ~10 mA - 2: ~20 mA - 3: ~40 mA The default value is 2. The drive strength of the internal pull-up (wpu) and pull-down (wpd) is ~75 µA. |
| 9 | Column“At Reset” in Table IO_MUX lists the status of each pin during reset, including input-enable (ie=1), internal pull-up (wpu) and internal pull-down (wpd). During reset, all pins are output-disabled. |
| 10 | Column “After Reset” in Table IO_MUX lists the status of each pin immediately after reset, including input-enable (ie=1), internal pull-up (wpu) and internal pull-down (wpd). After reset, each pin is set to “Function 0”. The output-enable is controlled by digital Function 0. |
| 11 | Table Ethernet_MAC is about the signal mapping inside Ethernet MAC. The Ethernet MAC supports MII and RMII interfaces, and supports both the internal PLL clock and the external clock source. For the MII interface, the Ethernet MAC is with/without the TX_ERR signal. MDC, MDIO, CRS and COL are slow signals, and can be mapped onto any GPIO pin through the GPIO-Matrix. |
| 12 | Table GPIO Matrix is for the GPIO-Matrix. The signals of the on-chip functional modules can be mapped onto any GPIO pin. Some signals can be mapped onto a pin by both IO-MUX and GPIO-Matrix, as shown in the column tagged as“Same input signal from IO_MUX core” in Table GPIO Matrix. |

| No. | Description |
| --- | --- |
| 13 | *In Table GPIO_Matrix，the column“Default Value if unassigned”records the default value of the an input signal if no GPIO is assigned to it. The actual value is determined by register GPIO_FUNCm_IN_INV_SEL and GPIO_FUNCm_IN_SEL. (The value of m ranges from 1 to 255.) |

## GPIO_Matrix

Table 6-2. GPIO_Matrix

| Signal No. | Input Signals | Default Value If Unassigned* | Same Input Signal from IO_MUX Core | Output Signals | Output Enabl Output Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | SPICLK_in | 0 | yes | SPICLK_out | SPICLK_oe |
| 1 | SPIQ_in | 0 | yes | SPIQ_out | SPIQ_oe |
| 2 | SPID_in | 0 | yes | SPID_out | SPID_oe |
| 3 | SPIHD_in | 0 | yes | SPIHD_out | SPIHD_oe |
| 4 | SPIWP_in | 0 | yes | SPIWP_out | SPIWP_oe |
| 5 | SPICS0_in | 0 | yes | SPICS0_out | SPICS0_oe |
| 6 | SPICS1_in | 0 | no | SPICS1_out | SPICS1_oe |
| 7 | SPICS2_in | 0 | no | SPICS2_out | SPICS2_oe |
| 8 | HSPICLK_in | 0 | yes | HSPICLK_out | HSPICLK_oe |
| 9 | HSPIQ_in | 0 | yes | HSPIQ_out | HSPIQ_oe |
| 10 | HSPID_in | 0 | yes | HSPID_out | HSPID_oe |
| 11 | HSPICS0_in | 0 | yes | HSPICS0_out | HSPICS0_oe |
| 12 | HSPIHD_in | 0 | yes | HSPIHD_out | HSPIHD_oe |
| 13 | HSPIWP_in | 0 | yes | HSPIWP_out | HSPIWP_oe |
| 14 | U0RXD_in | 0 | yes | U0TXD_out | 1’d1 |
| 15 | U0CTS_in | 0 | yes | U0RTS_out | 1’d1 |
| 16 | U0DSR_in | 0 | no | U0DTR_out | 1’d1 |
| 17 | U1RXD_in | 0 | yes | U1TXD_out | 1’d1 |
| 18 | U1CTS_in | 0 | yes | U1RTS_out | 1’d1 |
| 23 | I2S0O_BCK_in | 0 | no | I2S0O_BCK_out | 1’d1 |
| 24 | I2S1O_BCK_in | 0 | no | I2S1O_BCK_out | 1’d1 |
| 25 | I2S0O_WS_in | 0 | no | I2S0O_WS_out | 1’d1 |
| 26 | I2S1O_WS_in | 0 | no | I2S1O_WS_out | 1’d1 |
| 27 | I2S0I_BCK_in | 0 | no | I2S0I_BCK_out | 1’d1 |
| 28 | I2S0I_WS_in | 0 | no | I2S0I_WS_out | 1’d1 |
| 29 | I2CEXT0_SCL_in | 1 | no | I2CEXT0_SCL_out | 1’d1 |
| 30 | I2CEXT0_SDA_in | 1 | no | I2CEXT0_SDA_out | 1’d1 |
| 31 | pwm0_sync0_in | 0 | no | sdio_tohost_int_out | 1’d1 |
| 32 | pwm0_sync1_in | 0 | no | pwm0_out0a | 1’d1 |
| 33 | pwm0_sync2_in | 0 | no | pwm0_out0b | 1’d1 |

| Signal No. | Input Signals | Default Value If Unassigned* | Same Input Signal from IO_MUX Core | Output Signals | Output Enabl Output Signal |
| --- | --- | --- | --- | --- | --- |
| 34 | pwm0_f0_in | 0 | no | pwm0_out1a | 1’d1 |
| 35 | pwm0_f1_in | 0 | no | pwm0_out1b | 1’d1 |
| 36 | pwm0_f2_in | 0 | no | pwm0_out2a | 1’d1 |
| 37 | — | 0 | no | pwm0_out2b | 1’d1 |
| 39 | pcnt_sig_ch0_in0 | 0 | no | — | 1’d1 |
| 40 | pcnt_sig_ch1_in0 | 0 | no | — | 1’d1 |
| 41 | pcnt_ctrl_ch0_in0 | 0 | no | — | 1’d1 |
| 42 | pcnt_ctrl_ch1_in0 | 0 | no | — | 1’d1 |
| 43 | pcnt_sig_ch0_in1 | 0 | no | — | 1’d1 |
| 44 | pcnt_sig_ch1_in1 | 0 | no | — | 1’d1 |
| 45 | pcnt_ctrl_ch0_in1 | 0 | no | — | 1’d1 |
| 46 | pcnt_ctrl_ch1_in1 | 0 | no | — | 1’d1 |
| 47 | pcnt_sig_ch0_in2 | 0 | no | — | 1’d1 |
| 48 | pcnt_sig_ch1_in2 | 0 | no | — | 1’d1 |
| 49 | pcnt_ctrl_ch0_in2 | 0 | no | — | 1’d1 |
| 50 | pcnt_ctrl_ch1_in2 | 0 | no | — | 1’d1 |
| 51 | pcnt_sig_ch0_in3 | 0 | no | — | 1’d1 |
| 52 | pcnt_sig_ch1_in3 | 0 | no | — | 1’d1 |
| 53 | pcnt_ctrl_ch0_in3 | 0 | no | — | 1’d1 |
| 54 | pcnt_ctrl_ch1_in3 | 0 | no | — | 1’d1 |
| 55 | pcnt_sig_ch0_in4 | 0 | no | — | 1’d1 |
| 56 | pcnt_sig_ch1_in4 | 0 | no | — | 1’d1 |
| 57 | pcnt_ctrl_ch0_in4 | 0 | no | — | 1’d1 |
| 58 | pcnt_ctrl_ch1_in4 | 0 | no | — | 1’d1 |
| 61 | HSPICS1_in | 0 | no | HSPICS1_out | HSPICS1_oe |
| 62 | HSPICS2_in | 0 | no | HSPICS2_out | HSPICS2_oe |
| 63 | VSPICLK_in | 0 | yes | VSPICLK_out_mux | VSPICLK_oe |
| 64 | VSPIQ_in | 0 | yes | VSPIQ_out | VSPIQ_oe |
| 65 | VSPID_in | 0 | yes | VSPID_out | VSPID_oe |
| 66 | VSPIHD_in | 0 | yes | VSPIHD_out | VSPIHD_oe |
| 67 | VSPIWP_in | 0 | yes | VSPIWP_out | VSPIWP_oe |
| 68 | VSPICS0_in | 0 | yes | VSPICS0_out | VSPICS0_oe |
| 69 | VSPICS1_in | 0 | no | VSPICS1_out | VSPICS1_oe |
| 70 | VSPICS2_in | 0 | no | VSPICS2_out | VSPICS2_oe |
| 71 | pcnt_sig_ch0_in5 | 0 | no | ledc_hs_sig_out0 | 1’d1 |
| 72 | pcnt_sig_ch1_in5 | 0 | no | ledc_hs_sig_out1 | 1’d1 |
| 73 | pcnt_ctrl_ch0_in5 | 0 | no | ledc_hs_sig_out2 | 1’d1 |
| 74 | pcnt_ctrl_ch1_in5 | 0 | no | ledc_hs_sig_out3 | 1’d1 |
| 75 | pcnt_sig_ch0_in6 | 0 | no | ledc_hs_sig_out4 | 1’d1 |
| 76 | pcnt_sig_ch1_in6 | 0 | no | ledc_hs_sig_out5 | 1’d1 |

| Signal No. | Input Signals | Default Value If Unassigned* | Same Input Signal from IO_MUX Core | Output Signals | Output Enabl Output Signal |
| --- | --- | --- | --- | --- | --- |
| 77 | pcnt_ctrl_ch0_in6 | 0 | no | ledc_hs_sig_out6 | 1’d1 |
| 78 | pcnt_ctrl_ch1_in6 | 0 | no | ledc_hs_sig_out7 | 1’d1 |
| 79 | pcnt_sig_ch0_in7 | 0 | no | ledc_ls_sig_out0 | 1’d1 |
| 80 | pcnt_sig_ch1_in7 | 0 | no | ledc_ls_sig_out1 | 1’d1 |
| 81 | pcnt_ctrl_ch0_in7 | 0 | no | ledc_ls_sig_out2 | 1’d1 |
| 82 | pcnt_ctrl_ch1_in7 | 0 | no | ledc_ls_sig_out3 | 1’d1 |
| 83 | rmt_sig_in0 | 0 | no | ledc_ls_sig_out4 | 1’d1 |
| 84 | rmt_sig_in1 | 0 | no | ledc_ls_sig_out5 | 1’d1 |
| 85 | rmt_sig_in2 | 0 | no | ledc_ls_sig_out6 | 1’d1 |
| 86 | rmt_sig_in3 | 0 | no | ledc_ls_sig_out7 | 1’d1 |
| 87 | rmt_sig_in4 | 0 | no | rmt_sig_out0 | 1’d1 |
| 88 | rmt_sig_in5 | 0 | no | rmt_sig_out1 | 1’d1 |
| 89 | rmt_sig_in6 | 0 | no | rmt_sig_out2 | 1’d1 |
| 90 | rmt_sig_in7 | 0 | no | rmt_sig_out3 | 1’d1 |
| 91 | — | — | — | rmt_sig_out4 | 1’d1 |
| 92 | — | — | — | rmt_sig_out6 | 1’d1 |
| 94 | twai_rx | 1 | no | rmt_sig_out7 | 1’d1 |
| 95 | I2CEXT1_SCL_in | 1 | no | I2CEXT1_SCL_out | 1’d1 |
| 96 | I2CEXT1_SDA_in | 1 | no | I2CEXT1_SDA_out | 1’d1 |
| 97 | host_card_detect_n_1 | 0 | no | host_ccmd_od_pullup_en_n | 1’d1 |
| 98 | host_card_detect_n_2 | 0 | no | host_rst_n_1 | 1’d1 |
| 99 | host_card_write_prt_1 | 0 | no | host_rst_n_2 | 1’d1 |
| 100 | host_card_write_prt_2 | 0 | no | gpio_sd0_out | 1’d1 |
| 101 | host_card_int_n_1 | 0 | no | gpio_sd1_out | 1’d1 |
| 102 | host_card_int_n_2 | 0 | no | gpio_sd2_out | 1’d1 |
| 103 | pwm1_sync0_in | 0 | no | gpio_sd3_out | 1’d1 |
| 104 | pwm1_sync1_in | 0 | no | gpio_sd4_out | 1’d1 |
| 105 | pwm1_sync2_in | 0 | no | gpio_sd5_out | 1’d1 |
| 106 | pwm1_f0_in | 0 | no | gpio_sd6_out | 1’d1 |
| 107 | pwm1_f1_in | 0 | no | gpio_sd7_out | 1’d1 |
| 108 | pwm1_f2_in | 0 | no | pwm1_out0a | 1’d1 |
| 109 | pwm0_cap0_in | 0 | no | pwm1_out0b | 1’d1 |
| 110 | pwm0_cap1_in | 0 | no | pwm1_out1a | 1’d1 |
| 111 | pwm0_cap2_in | 0 | no | pwm1_out1b | 1’d1 |
| 112 | pwm1_cap0_in | 0 | no | pwm1_out2a | 1’d1 |
| 113 | pwm1_cap1_in | 0 | no | pwm1_out2b | 1’d1 |
| 114 | pwm1_cap2_in | 0 | no | pwm2_out1h | 1’d1 |
| 115 | pwm2_flta | 1 | no | pwm2_out1l | 1’d1 |
| 116 | pwm2_fltb | 1 | no | pwm2_out2h | 1’d1 |
| 117 | pwm2_cap1_in | 0 | no | pwm2_out2l | 1’d1 |

| Signal No. | Input Signals | Default Value If Unassigned* | Same Input Signal from IO_MUX Core | Output Signals | Output Enabl Output Signal |
| --- | --- | --- | --- | --- | --- |
| 118 | pwm2_cap2_in | 0 | no | pwm2_out3h | 1’d1 |
| 119 | pwm2_cap3_in | 0 | no | pwm2_out3l | 1’d1 |
| 120 | pwm3_flta | 1 | no | pwm2_out4h | 1’d1 |
| 121 | pwm3_fltb | 1 | no | pwm2_out4l | 1’d1 |
| 122 | pwm3_cap1_in | 0 | no | — | 1’d1 |
| 123 | pwm3_cap2_in | 0 | no | twai_tx | 1’d1 |
| 124 | pwm3_cap3_in | 0 | no | twai_bus_off_on | 1’d1 |
| 125 | — | — | — | twai_clkout | 1’d1 |
| 140 | I2S0I_DATA_in0 | 0 | no | I2S0O_DATA_out0 | 1’d1 |
| 141 | I2S0I_DATA_in1 | 0 | no | I2S0O_DATA_out1 | 1’d1 |
| 142 | I2S0I_DATA_in2 | 0 | no | I2S0O_DATA_out2 | 1’d1 |
| 143 | I2S0I_DATA_in3 | 0 | no | I2S0O_DATA_out3 | 1’d1 |
| 144 | I2S0I_DATA_in4 | 0 | no | I2S0O_DATA_out4 | 1’d1 |
| 145 | I2S0I_DATA_in5 | 0 | no | I2S0O_DATA_out5 | 1’d1 |
| 146 | I2S0I_DATA_in6 | 0 | no | I2S0O_DATA_out6 | 1’d1 |
| 147 | I2S0I_DATA_in7 | 0 | no | I2S0O_DATA_out7 | 1’d1 |
| 148 | I2S0I_DATA_in8 | 0 | no | I2S0O_DATA_out8 | 1’d1 |
| 149 | I2S0I_DATA_in9 | 0 | no | I2S0O_DATA_out9 | 1’d1 |
| 150 | I2S0I_DATA_in10 | 0 | no | I2S0O_DATA_out10 | 1’d1 |
| 151 | I2S0I_DATA_in11 | 0 | no | I2S0O_DATA_out11 | 1’d1 |
| 152 | I2S0I_DATA_in12 | 0 | no | I2S0O_DATA_out12 | 1’d1 |
| 153 | I2S0I_DATA_in13 | 0 | no | I2S0O_DATA_out13 | 1’d1 |
| 154 | I2S0I_DATA_in14 | 0 | no | I2S0O_DATA_out14 | 1’d1 |
| 155 | I2S0I_DATA_in15 | 0 | no | I2S0O_DATA_out15 | 1’d1 |
| 156 | — | — | — | I2S0O_DATA_out16 | 1’d1 |
| 157 | — | — | — | I2S0O_DATA_out17 | 1’d1 |
| 158 | — | — | — | I2S0O_DATA_out18 | 1’d1 |
| 159 | — | — | — | I2S0O_DATA_out19 | 1’d1 |
| 160 | — | — | — | I2S0O_DATA_out20 | 1’d1 |
| 161 | — | — | — | I2S0O_DATA_out21 | 1’d1 |
| 162 | — | — | — | I2S0O_DATA_out22 | 1’d1 |
| 163 | — | — | — | I2S0O_DATA_out23 | 1’d1 |
| 164 | I2S1I_BCK_in | 0 | no | I2S1I_BCK_out | 1’d1 |
| 165 | I2S1I_WS_in | 0 | no | I2S1I_WS_out | 1’d1 |
| 166 | I2S1I_DATA_in0 | 0 | no | I2S1O_DATA_out0 | 1’d1 |
| 167 | I2S1I_DATA_in1 | 0 | no | I2S1O_DATA_out1 | 1’d1 |
| 168 | I2S1I_DATA_in2 | 0 | no | I2S1O_DATA_out2 | 1’d1 |
| 169 | I2S1I_DATA_in3 | 0 | no | I2S1O_DATA_out3 | 1’d1 |
| 170 | I2S1I_DATA_in4 | 0 | no | I2S1O_DATA_out4 | 1’d1 |
| 171 | I2S1I_DATA_in5 | 0 | no | I2S1O_DATA_out5 | 1’d1 |

| Signal No. | Input Signals | Default Value If Unassigned* | Same Input Signal from IO_MUX Core | Output Signals | Output Enabl Output Signal |
| --- | --- | --- | --- | --- | --- |
| 172 | I2S1I_DATA_in6 | 0 | no | I2S1O_DATA_out6 | 1’d1 |
| 173 | I2S1I_DATA_in7 | 0 | no | I2S1O_DATA_out7 | 1’d1 |
| 174 | I2S1I_DATA_in8 | 0 | no | I2S1O_DATA_out8 | 1’d1 |
| 175 | I2S1I_DATA_in9 | 0 | no | I2S1O_DATA_out9 | 1’d1 |
| 176 | I2S1I_DATA_in10 | 0 | no | I2S1O_DATA_out10 | 1’d1 |
| 177 | I2S1I_DATA_in11 | 0 | no | I2S1O_DATA_out11 | 1’d1 |
| 178 | I2S1I_DATA_in12 | 0 | no | I2S1O_DATA_out12 | 1’d1 |
| 179 | I2S1I_DATA_in13 | 0 | no | I2S1O_DATA_out13 | 1’d1 |
| 180 | I2S1I_DATA_in14 | 0 | no | I2S1O_DATA_out14 | 1’d1 |
| 181 | I2S1I_DATA_in15 | 0 | no | I2S1O_DATA_out15 | 1’d1 |
| 182 | — | — | — | I2S1O_DATA_out16 | 1’d1 |
| 183 | — | — | — | I2S1O_DATA_out17 | 1’d1 |
| 184 | — | — | — | I2S1O_DATA_out18 | 1’d1 |
| 185 | — | — | — | I2S1O_DATA_out19 | 1’d1 |
| 186 | — | — | — | I2S1O_DATA_out20 | 1’d1 |
| 187 | — | — | — | I2S1O_DATA_out21 | 1’d1 |
| 188 | — | — | — | I2S1O_DATA_out22 | 1’d1 |
| 189 | — | — | — | I2S1O_DATA_out23 | 1’d1 |
| 190 | I2S0I_H_SYNC | 0 | no | pwm3_out1h | 1’d1 |
| 191 | I2S0I_V_SYNC | 0 | no | pwm3_out1l | 1’d1 |
| 192 | I2S0I_H_ENABLE | 0 | no | pwm3_out2h | 1’d1 |
| 193 | I2S1I_H_SYNC | 0 | no | pwm3_out2l | 1’d1 |
| 194 | I2S1I_V_SYNC | 0 | no | pwm3_out3h | 1’d1 |
| 195 | I2S1I_H_ENABLE | 0 | no | pwm3_out3l | 1’d1 |
| 196 | — | — | — | pwm3_out4h | 1’d1 |
| 197 | — | — | — | pwm3_out4l | 1’d1 |
| 198 | U2RXD_in | 0 | yes | U2TXD_out | 1’d1 |
| 199 | U2CTS_in | 0 | yes | U2RTS_out | 1’d1 |
| 200 | emac_mdc_i | 0 | no | emac_mdc_o | emac_mdc_o |
| 201 | emac_mdi_i | 0 | no | emac_mdo_o | emac_mdo_o |
| 202 | emac_crs_i | 0 | no | emac_crs_o | emac_crs_oe |
| 203 | emac_col_i | 0 | no | emac_col_o | emac_col_oe |
| 204 | pcmfsync_in | 0 | no | bt_audio0_irq | 1’d1 |
| 205 | pcmclk_in | 0 | no | bt_audio1_irq | 1’d1 |
| 206 | pcmdin | 0 | no | bt_audio2_irq | 1’d1 |
| 207 | — | — | — | ble_audio0_irq | 1’d1 |
| 208 | — | — | — | ble_audio1_irq | 1’d1 |
| 209 | — | — | — | ble_audio2_irq | 1’d1 |
| 210 | — | — | — | pcmfsync_out | pcmfsync_en |
| 211 | — | — | — | pcmclk_out | pcmclk_en |

| Signal No. | Input Signals | Default Value If Unassigned* | Same Input Signal from IO_MUX Core | Output Signals | Output Enabl Output Signal |
| --- | --- | --- | --- | --- | --- |
| 212 | — | — | — | pcmdout | pcmdout_en |
| 213 | — | — | — | ble_audio_sync0_p | 1’d1 |
| 214 | — | — | — | ble_audio_sync1_p | 1’d1 |
| 215 | — | — | — | ble_audio_sync2_p | 1’d1 |
| 224 | — | — | — | sig_in_func224 | 1’d1 |
| 225 | — | — | — | sig_in_func225 | 1’d1 |
| 226 | — | — | — | sig_in_func226 | 1’d1 |
| 227 | — | — | — | sig_in_func227 | 1’d1 |
| 228 | — | — | — | sig_in_func228 | 1’d1 |

## Ethernet_MAC

Table 6-3. Ethernet_MAC

| Pin Name | Function6 | MII (int_osc) | MII (ext_osc) | RMII (int_osc) | RMII (ext_osc) |
| --- | --- | --- | --- | --- | --- |
| GPIO0 | EMAC_TX_CLK | TX_CLK (I) | TX_CLK (I) | CLK_OUT(O) | EXT_OSC_CLK(I) |
| GPIO5 | EMAC_RX_CLK | RX_CLK (I) | RX_CLK (I) | — | — |
| GPIO21 | EMAC_TX_EN | TX_EN(O) | TX_EN(O) | TX_EN(O) | TX_EN(O) |
| GPIO19 | EMAC_TXD0 | TXD[0](O) | TXD[0](O) | TXD[0](O) | TXD[0](O) |
| GPIO22 | EMAC_TXD1 | TXD[1](O) | TXD[1](O) | TXD[1](O) | TXD[1](O) |
| MTMS | EMAC_TXD2 | TXD[2](O) | TXD[2](O) | — | — |
| MTDI | EMAC_TXD3 | TXD[3](O) | TXD[3](O) | — | — |
| MTCK | EMAC_RX_ER | RX_ER(I) | RX_ER(I) | — | — |
| GPIO27 | EMAC_RX_DV | RX_DV(I) | RX_DV(I) | CRS_DV(I) | CRS_DV(I) |
| GPIO25 | EMAC_RXD0 | RXD[0](I) | RXD[0](I) | RXD[0](I) | RXD[0](I) |
| GPIO26 | EMAC_RXD1 | RXD[1](I) | RXD[1](I) | RXD[1](I) | RXD[1](I) |
| U0TXD | EMAC_RXD2 | RXD[2](I) | RXD[2](I) | — | — |
| MTDO | EMAC_RXD3 | RXD[3](I) | RXD[3](I) | — | — |
| GPIO16 | EMAC_CLK_OUT | CLK_OUT(O) | — | CLK_OUT(O) | — |
| GPIO17 | EMAC_CLK_OUT_180 | CLK_OUT_180(O) | — | CLK_OUT_180(O) | — |
| GPIO4 | EMAC_TX_ER | TX_ERR(O)* | TX_ERR(O)* | — | — |
| In GPIO Matrix* | — | MDC(O) | MDC(O) | MDC(O) | MDC(O) |
| In GPIO Matrix* | — | MDIO(IO) | MDIO(IO) | MDIO(IO) | MDIO(IO) |
| In GPIO Matrix* | — | CRS(I) | CRS(I) | — | — |
| In GPIO Matrix* | — | COL(I) | COL(I) | — | — |
| *Notes: 1. The GPIO Matrix can be any GPIO. 2. The TX_ERR (O) is optional. |  |  |  |  |  |

## IO_MUX

For the list of IO_MUX pins, please see the next page.

Appendix A

Espressif Systems

70

Submit Documentation Feedback

ESP32 Series Datasheet v5.2

IO_MUX

| Pin No. | Power Supply Pin | Analog Pin | Digital Pin | Power Domain | Analog Function0 | Analog Function1 | Analog Function2 | RTC Function0 | RTC Function1 | Function0 | Type | Function1 | Type | Function2 | Type | Function3 | Type | Function4 | Type | Function5 | Type | Drive Strength (2’d2: 20 mA) | At Reset | After Reset |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | VDDA |  |  | VDDA supply in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | LNA_IN |  | VDD3P3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | VDD3P3 |  |  | VDD3P3 supply in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | VDD3P3 |  |  | VDD3P3 supply in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | SENSOR_VP |  | VDD3P3_RTC |  | ADC1_CH0 |  | RTC_GPIO0 |  | GPIO36 | I |  |  | GPIO36 | I |  |  |  |  |  |  |  | oe=0, ie=0 | oe=0, ie=0 |
| 6 |  | SENSOR_CAPP |  | VDD3P3_RTC |  | ADC1_CH1 |  | RTC_GPIO1 |  | GPIO37 | I |  |  | GPIO37 | I |  |  |  |  |  |  |  | oe=0, ie=0 | oe=0, ie=0 |
| 7 |  | SENSOR_CAPN |  | VDD3P3_RTC |  | ADC1_CH2 |  | RTC_GPIO2 |  | GPIO38 | I |  |  | GPIO38 | I |  |  |  |  |  |  |  | oe=0, ie=0 | oe=0, ie=0 |
| 8 |  | SENSOR_VN |  | VDD3P3_RTC |  | ADC1_CH3 |  | RTC_GPIO3 |  | GPIO39 | I |  |  | GPIO39 | I |  |  |  |  |  |  |  | oe=0, ie=0 | oe=0, ie=0 |
| 9 |  | CHIP_PU |  | VDD3P3_RTC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  | VDET_1 |  | VDD3P3_RTC |  | ADC1_CH6 |  | RTC_GPIO4 |  | GPIO34 | I |  |  | GPIO34 | I |  |  |  |  |  |  |  | oe=0, ie=0 | oe=0, ie=0 |
| 11 |  | VDET_2 |  | VDD3P3_RTC |  | ADC1_CH7 |  | RTC_GPIO5 |  | GPIO35 | I |  |  | GPIO35 | I |  |  |  |  |  |  |  | oe=0, ie=0 | oe=0, ie=0 |
| 12 |  | 32K_XP |  | VDD3P3_RTC | XTAL_32K_P | ADC1_CH4 | TOUCH9 | RTC_GPIO9 |  | GPIO32 | I/O/T |  |  | GPIO32 | I/O/T |  |  |  |  |  |  | 2'd2 | oe=0, ie=0 | oe=0, ie=0 |
| 13 |  | 32K_XN |  | VDD3P3_RTC | XTAL_32K_N | ADC1_CH5 | TOUCH8 | RTC_GPIO8 |  | GPIO33 | I/O/T |  |  | GPIO33 | I/O/T |  |  |  |  |  |  | 2'd2 | oe=0, ie=0 | oe=0, ie=0 |
| 14 |  |  | GPIO25 | VDD3P3_RTC | DAC_1 | ADC2_CH8 |  | RTC_GPIO6 |  | GPIO25 | I/O/T |  |  | GPIO25 | I/O/T |  |  |  |  | EMAC_RXD0 | I | 2'd2 | oe=0, ie=0 | oe=0, ie=0 |
| 15 |  |  | GPIO26 | VDD3P3_RTC | DAC_2 | ADC2_CH9 |  | RTC_GPIO7 |  | GPIO26 | I/O/T |  |  | GPIO26 | I/O/T |  |  |  |  | EMAC_RXD1 | I | 2'd2 | oe=0, ie=0 | oe=0, ie=0 |
| 16 |  |  | GPIO27 | VDD3P3_RTC |  | ADC2_CH7 | TOUCH7 | RTC_GPIO17 |  | GPIO27 | I/O/T |  |  | GPIO27 | I/O/T |  |  |  |  | EMAC_RX_DV | I | 2'd2 | oe=0, ie=0 | oe=0, ie=0 |
| 17 |  |  | MTMS | VDD3P3_RTC |  | ADC2_CH6 | TOUCH6 | RTC_GPIO16 |  | MTMS | I0 | HSPICLK | I/O/T | GPIO14 | I/O/T | HS2_CLK | O | SD_CLK | I0 | EMAC_TXD2 | O | 2'd2 | oe=0, ie=0 | oe=0, ie=1, wpu |
| 18 |  |  | MTDI | VDD3P3_RTC |  | ADC2_CH5 | TOUCH5 | RTC_GPIO15 |  | MTDI | I1 | HSPIQ | I/O/T | GPIO12 | I/O/T | HS2_DATA2 | I1/O/T | SD_DATA2 | I1/O/T | EMAC_TXD3 | O | 2'd2 | oe=0, ie=1, wpd | oe=0, ie=1, wpd |
| 19 | VDD3P3_RTC |  |  | VDD3P3_RTC supply in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  | MTCK | VDD3P3_RTC |  | ADC2_CH4 | TOUCH4 | RTC_GPIO14 |  | MTCK | I1 | HSPID | I/O/T | GPIO13 | I/O/T | HS2_DATA3 | I1/O/T | SD_DATA3 | I1/O/T | EMAC_RX_ER | I | 2'd2 | oe=0, ie=0 | oe=0, ie=1, wpd |
| 21 |  |  | MTDO | VDD3P3_RTC |  | ADC2_CH3 | TOUCH3 | RTC_GPIO13 | I2C_SDA | MTDO | O/T | HSPICS0 | I/O/T | GPIO15 | I/O/T | HS2_CMD | I1/O/T | SD_CMD | I1/O/T | EMAC_RXD3 | I | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 22 |  |  | GPIO2 | VDD3P3_RTC |  | ADC2_CH2 | TOUCH2 | RTC_GPIO12 | I2C_SCL | GPIO2 | I/O/T | HSPIWP | I/O/T | GPIO2 | I/O/T | HS2_DATA0 | I1/O/T | SD_DATA0 | I1/O/T |  |  | 2'd2 | oe=0, ie=1, wpd | oe=0, ie=1, wpd |
| 23 |  |  | GPIO0 | VDD3P3_RTC |  | ADC2_CH1 | TOUCH1 | RTC_GPIO11 | I2C_SDA | GPIO0 | I/O/T | CLK_OUT1 | O | GPIO0 | I/O/T |  |  |  |  | EMAC_TX_CLK | I | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 24 |  |  | GPIO4 | VDD3P3_RTC |  | ADC2_CH0 | TOUCH0 | RTC_GPIO10 | I2C_SCL | GPIO4 | I/O/T | HSPIHD | I/O/T | GPIO4 | I/O/T | HS2_DATA1 | I1/O/T | SD_DATA1 | I1/O/T | EMAC_TX_ER | O | 2'd2 | oe=0, ie=1, wpd | oe=0, ie=1, wpd |
| 25 |  |  | GPIO16 | VDD_SDIO |  |  |  |  |  | GPIO16 | I/O/T |  |  | GPIO16 | I/O/T | HS1_DATA4 | I1/O/T | U2RXD | I1 | EMAC_CLK_OUT | O | 2'd2 | oe=0, ie=0 | oe=0, ie=1 |
| 26 | VDD_SDIO |  |  | VDD_SDIO supply out/in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  | GPIO17 | VDD_SDIO |  |  |  |  |  | GPIO17 | I/O/T |  |  | GPIO17 | I/O/T | HS1_DATA5 | I1/O/T | U2TXD | O | EMAC_CLK_OUT_180 | O | 2'd2 | oe=0, ie=0 | oe=0, ie=1 |
| 28 |  |  | SD_DATA_2 | VDD_SDIO |  |  |  |  |  | SD_DATA2 | I1/O/T | SPIHD | I/O/T | GPIO9 | I/O/T | HS1_DATA2 | I1/O/T | U1RXD | I1 |  |  | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 29 |  |  | SD_DATA_3 | VDD_SDIO |  |  |  |  |  | SD_DATA3 | I0/O/T | SPIWP | I/O/T | GPIO10 | I/O/T | HS1_DATA3 | I1/O/T | U1TXD | O |  |  | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 30 |  |  | SD_CMD | VDD_SDIO |  |  |  |  |  | SD_CMD | I1/O/T | SPICS0 | I/O/T | GPIO11 | I/O/T | HS1_CMD | I1/O/T | U1RTS | O |  |  | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 31 |  |  | SD_CLK | VDD_SDIO |  |  |  |  |  | SD_CLK | I0 | SPICLK | I/O/T | GPIO6 | I/O/T | HS1_CLK | O | U1CTS | I1 |  |  | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 32 |  |  | SD_DATA_0 | VDD_SDIO |  |  |  |  |  | SD_DATA0 | I1/O/T | SPIQ | I/O/T | GPIO7 | I/O/T | HS1_DATA0 | I1/O/T | U2RTS | O |  |  | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 33 |  |  | SD_DATA_1 | VDD_SDIO |  |  |  |  |  | SD_DATA1 | I1/O/T | SPID | I/O/T | GPIO8 | I/O/T | HS1_DATA1 | I1/O/T | U2CTS | I1 |  |  | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 34 |  |  | GPIO5 | VDD3P3_CPU |  |  |  |  |  | GPIO5 | I/O/T | VSPICS0 | I/O/T | GPIO5 | I/O/T | HS1_DATA6 | I1/O/T |  |  | EMAC_RX_CLK | I | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 35 |  |  | GPIO18 | VDD3P3_CPU |  |  |  |  |  | GPIO18 | I/O/T | VSPICLK | I/O/T | GPIO18 | I/O/T | HS1_DATA7 | I1/O/T |  |  |  |  | 2'd2 | oe=0, ie=0 | oe=0, ie=1 |
| 36 |  |  | GPIO23 | VDD3P3_CPU |  |  |  |  |  | GPIO23 | I/O/T | VSPID | I/O/T | GPIO23 | I/O/T | HS1_STROBE | I0 |  |  |  |  | 2'd2 | oe=0, ie=0 | oe=0, ie=1 |
| 37 | VDD3P3_CPU |  |  | VDD3P3_CPU supply in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  | GPIO19 | VDD3P3_CPU |  |  |  |  |  | GPIO19 | I/O/T | VSPIQ | I/O/T | GPIO19 | I/O/T | U0CTS | I1 |  |  | EMAC_TXD0 | O | 2'd2 | oe=0, ie=0 | oe=0, ie=1 |
| 39 |  |  | GPIO22 | VDD3P3_CPU |  |  |  |  |  | GPIO22 | I/O/T | VSPIWP | I/O/T | GPIO22 | I/O/T | U0RTS | O |  |  | EMAC_TXD1 | O | 2'd2 | oe=0, ie=0 | oe=0, ie=1 |
| 40 |  |  | U0RXD | VDD3P3_CPU |  |  |  |  |  | U0RXD | I1 | CLK_OUT2 | O | GPIO3 | I/O/T |  |  |  |  |  |  | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 41 |  |  | U0TXD | VDD3P3_CPU |  |  |  |  |  | U0TXD | O | CLK_OUT3 | O | GPIO1 | I/O/T |  |  |  |  | EMAC_RXD2 | I | 2'd2 | oe=0, ie=1, wpu | oe=0, ie=1, wpu |
| 42 |  |  | GPIO21 | VDD3P3_CPU |  |  |  |  |  | GPIO21 | I/O/T | VSPIHD | I/O/T | GPIO21 | I/O/T |  |  |  |  | EMAC_TX_EN | O | 2'd2 | oe=0, ie=0 | oe=0, ie=1 |
| 43 | VDDA |  |  | VDDA supply in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 |  | XTAL_N |  | VDDA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 45 |  | XTAL_P |  | VDDA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 | VDDA |  |  | VDDA supply in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 |  | CAP2 |  | VDDA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 |  | CAP1 |  | VDDA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Number | 8 | 14 | 26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: - wpu: weak pull-up; - wpd: weak pull-down; - ie: input enable; - oe: output enable; - Please see Table: Notes on ESP32 Pin Lists for more information.（请参考表：管脚清单说明。） |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Revision History

| Date | Version | Release notes |
| --- | --- | --- |
| 2025.11 | v5.2 | - ESP32-D0WDR2-V3 is end of life and upgraded to ESP32-D0WDRH2-V3 - Table 1-1 Comparison: Updated ”Ordering Code” to ”Part Number” |
| 2025.10 | v5.1 | - Section 4.8.3 Universal Asynchronous Receiver Transmitter (UART): Added description ”Programmable baud rates up to 5 MBaud” - Section 3 Boot Configurations: Fixed the typo about Internal LDO (VDD_SDIO) Voltage Control - Fixed other typos |
| 2025.08 | v5.0 | - Table 2-3 Power Pins: Added power pin 1 VDDA - Updated Figure 3-1 Visualization of Timing Parameters for the Strapping Pins - Table 5-3 DC Characteristics (3.3 V, 25 °C): Added VIH_nRST |
| 2025.04 | v4.9 | - Section CPU and Memory: Improved CoreMark scores - Section 3.1 Chip Boot Mode Control: Modified the description from ”valid only for ESP32 ECO V3” to ”valid only for ESP32 chip revisions v3.0 and higher” - Section 4.8.2 Serial Peripheral Interface (SPI): Added information about SPI - Table 2-2 Analog Pins: Fixed typos about pin numbers |
| 2025.01 | v4.8 | - Section 3 Boot Configurations: Fixed the typo about JTAG signal source control - Section 2.2 Pin Overview: Added a note about JTAG interface signals - Table 2-5 Pin Mapping Between Chip and Flash/PSRAM: Modified a note about VDD_SDIO |
| 2024.09 | v4.7 | - Table 5-2 Recommended Power Supply Characteristics: Deleted a note about VDD3P3_RTC limitation - Section 4.1.1 CPU: Fixed the link to Cadence Xtensa ISA Summary - Section 4.8.7 Pulse Counter Controller (PCNT): Fixed the typo in the Feature List |
| 2024.08 | v4.6 | Improved the formatting, structure, and wording in the following sections: - Section 2 Pins - Section 3 Boot Configurations (used to be named as ”Strapping Pins”) - Section 4 Functional Description |

Cont’d on next page

Cont’d from previous page

| Date | Version | Release notes |
| --- | --- | --- |
| 2024.02 | v4.5 | Section 2.5.3 Chip Power-up and Reset: Updated the link to the VDD_SDIO 1.8 V circuit design to ESP32 Hardware Design Guidelines |
| 2023.12 | v4.4 | Table 1-1 Comparison: Added information about flash under the table |
| 2023.07 | v4.3 | - Updated formatting throughout the document - Updated wording in some sections - Added a new section 2.3.1 Restrictions for GPIOs and RTC_GPIOs - Added a new section 4.1.5 Cache |
| 2023.01 | v4.2 | - Removed contents about hall sensor according to PCN20221202 - Section 4.9.3 Touch Sensor: Added a note about limited applications of touch sensor |
| 2022.12 | v4.1 | - Section 4.1.1 CPU: Added link to Xtensa® Instruction Set Architecture (ISA) Summary - Table 1-1 Comparison: Updated the description about chip revision upgrade |
| 2022.10 | v4.0 | - Section Product Overview: Updated the description - Table 2-6 Pin Mapping Between Chip and Flash/PSRAM: Added two notes below the table - Section 2.5.2 Power Scheme: Added a new item to “Notes on power supply” - Updated Figure 1-1 ESP32 Series Nomenclature - Table 1-1 Comparison: Added a new column “VDD_SDIO Voltage” - Section 4.8.12 TWAI® Controller: Updated the bit rates - Added Not Recommended for New Designs (NRND) label to ESP32-S0WD |
| 2022.03 | v3.9 | - Added a new chip variant ESP32-D0WDR2-V3 - Added Table 2-5 Pin Mapping Between Chip and Flash/PSRAM and Table 2-6 Pin Mapping Between Chip and Flash/PSRAM - Updated Figure 6-2 QFN48 (5×5 mm) Package - Updated Appendix IO_MUX - Updated Table 4-6 Peripheral Pin Configurations - Section 3.1 Chip Boot Mode Control: Added links to ESP32 Technical Reference Manual |

Cont’d on next page

Cont’d from previous page

| Date | Version | Release notes |
| --- | --- | --- |
| 2021.10 | v3.8 | - Upgraded ESP32-U4WDH variant from single-core to dual-core, see PCN-2021-021. The single-core version coexists with the new dual-core version around December 2, 2021. The physical product is subject to batch tracking. - Section CPU and Memory: Added CoreMark® score - Section 4.8.12 TWAI® Controller: Updated the description - Added Not Recommended for New Designs (NRND) label to the ESP32-D0WDQ6-V3 variant - Section 6 Packaging: Provided a link to Espressif Chip Package Information - Updated Section Bluetooth |
| 2021.07 | v3.7 | - Removed ESP32-D2WD variant - Section 4.7.1 Bluetooth Radio and Baseband: Updated wording - Updated pin function numbers starting from Function0 - Added Not Recommended for New Designs (NRND) label to ESP32-D0WD and ESP32-D0WDQ6 variants |
| 2021.03 | V3.6 | - Updated Figure Block Diagram - Updated Table 5-5 Reliability - Updated Figure 2-3 ESP32 Power Scheme - Updated Table 5-2 Recommended Power Supply Characteristics - Updated the notes below Table 2-4 Description of Timing Parameters for Power-up and Reset - Table 4-1, 4-6, Section 4.8.12 TWAI® Controller: Added more information about TWAI® |
| 2021.01 | V3.5 | - Table 2-1 Pin Overview: Updated the description for CAP2 from 3 nF to 3.3 nF - Section Advanced Peripheral Interfaces: Added TWAI® - Updated Figure Block Diagram - Appendix IO_MUX: Updated the reset values for MTCK, MTMS, GPIO27 |
| 2020.04 | V3.4 | - Added one chip variant: ESP32-U4WDH - Updated some figures in Table 4-2, 5-6, 5-7, 5-9, 5-11, 5-12 - Table 5-7 Receiver –Basic Data Rate: Added a note under the table |

Cont’d on next page

Cont’d from previous page

| Date | Version | Release notes |
| --- | --- | --- |
| 2020.01 | V3.3 | - Added two chip variants: ESP32-D0WD-V3 and ESP32-D0WDQ6-V3. - Added a note under Table 4-3 Analog-to-Digital Converter (ADC) |
| 2019.10 | V3.2 | - Updated Figure 2-4 Visualization of Timing Parameters for Power-up and Reset |
| 2019.07 | V3.1 | - Table 2-1 Pin Overview: Added pin-pin mapping between ESP32-D2WD and the in-package flash under the table - Updated Figure 1-1 ESP32 Series Nomenclature |
| 2019.04 | V3.0 | - Section 3 Boot Configurations (used to be named as ”Strapping Pins”): Added information about the setup and hold times for the strapping pins |
| 2019.02 | V2.9 | - Table 2-1 Pin Overview: Applied new formatting - Table 4-6 Peripheral Pin Configurations: Fixed typos with respect to the ADC1 channel mappings |
| 2019.01 | V2.8 | - Changed the RF power control range in Table 5-7, 5-10, and 5-12 from –12 ~ +12 to –12 ~ +9 dBm; - Small text changes |
| 2018.11 | V2.7 | - Updated Section Applications - Table IO_MUX: Updated pin statuses at reset and after reset |
| 2018.10 | V2.6 | - Section 6 Packaging: Updated QFN package drawings |
| 2018.08 | V2.5 | - Table 5-1 Absolute Maximum Ratings: Added ”Cumulative IO output current” - Table 5-3 DC Characteristics (3.3 V, 25 °C): Added more parameters - Appendix IO_MUX: Changed the power domain names to be consistent with the pin names |

Cont’d on next page

Cont’d from previous page

| Date | Version | Release notes |
| --- | --- | --- |
| 2018.07 | V2.4 | - Deleted information on Packet Traffic Arbitration (PTA); - Added Figure 2-4 Visualization of Timing Parameters for Power-up and Reset - Table 4-2 Power Management Unit (PMU): Added the current consumption figures for dual-core SoCs - Updated Section 4.9.1 Analog-to-Digital Converter (ADC) |
| 2018.06 | V2.3 | - Table 4-2 Power Management Unit (PMU): Added the current consumption figures at CPU frequency of 160 MHz |
| 2018.05 | V2.2 | - Table 2-1 Pin Overview: Changed the voltage range of VDD3P3_RTC from 1.8-3.6 V to 2.3-3.6 V - Updated Section 2.5.2 Power Scheme - Updated Section 4.1.3 External Flash and RAM - Updated Table 4-2 Power Management Unit (PMU) - Removed content about temperature sensor; Changes to electrical characteristics: - Updated Table 5-1 Absolute Maximum Ratings - Added Table 5-2 Recommended Power Supply Characteristics - Added Table 5-3 DC Characteristics (3.3 V, 25 °C) - Added Table 5-5 Reliability - Table 5-7 Receiver –Basic Data Rate: Updated the values of ”Gain control step” and ”Adjacent channel transmit power” - Table 5-10 Transmitter –Enhanced Data Rate: Updated the values of ”Gain control step”, ”π/4 DQPSK modulation accuracy”, ”8 DPSK modulation accuracy”, and ”In-band spurious emissions” - Table 5-12 Transmitter: Updated the values of ”Gain control step” and ”Adjacent channel transmit power” |
| 2018.01 | V2.1 | - Deleted software-specific features; - Deleted information on LNA pre-amplifier; - Specified the CPU speed and flash speed of ESP32-D2WD; - Section 2.5.2 Power Scheme: Added notes |
| 2017.12 | V2.0 | - Section 6 Packaging: Added a note on the sequence of pin number |

Cont’d on next page

Cont’d from previous page

| Date | Version | Release notes |
| --- | --- | --- |
| 2017.10 | V1.9 | - Table 2-1 Pin Overview: Updated the description of pin CHIP_PU - Section 2.5.2 Power Scheme: Added a note - Section 3 Boot Configurations (used to be named as ”Strapping Pins”): Updated the description of the chip’s system reset - Section 4.6.4 Wi-Fi Radio and Baseband: Added a description of antenna diversity and selection - Table 4-2 Power Management Unit (PMU): Deleted ”Association sleep pattern”, added notes to Active sleep and Modem-sleep |
| 2017.08 | V1.8 | - Added Table 4-6 Peripheral Pin Configurations - Figure Block Diagram: Corrected a typo |
| 2017.08 | V1.7 | - Section Bluetooth: Changed the transmitting power to +12 dBm; the sensitivity of NZIF receiver to –97 dBm - Table 2-1 Pin Overview: Added a note - section 4.1.1 CPU: Added 160 MHz clock frequency - Section 4.6.4 Wi-Fi Radio and Baseband: Changed the transmitting power from 21 dBm to 20.5 dBm - Section 4.7.1 Bluetooth Radio and Baseband: Changed the dynamic control range of class-1, class-2 and class-3 transmit output powers to ”up to 24 dBm”; changed the dynamic range of NZIF receiver sensitivity to ”over 97 dB” - Table 4-2 Power Management Unit (PMU): Added two notes - Updated Section 4.8.1 General Purpose Input / Output Interface (GPIO) - Updated Section 4.8.11 SDIO/SPI Slave Controller - Updated Table 5-1 Absolute Maximum Ratings - Table 5.4 RF Current Consumption in Active Mode: Changed the duty cycle on which the transmitters’measurements are based to 50%. - Table 5-6 Wi-Fi Radio: Added a note on “Output impedance” - Table 5-7, 5-9, 5-11: Updated parameter ”Sensitivity” - Table 5-7, 5-10, 5-12: Updated parameters ”RF transmit power” and ”RF power control range”; added parameter ”Gain control step” - Deleted Chapters: ”Touch Sensor” and ”Code Examples”; - Added a link to certification download. |
| 2017.06 | V1.6 | - Section Complete Integration Solution: Changed the number of external components to 20 - Section 4.8.1 General Purpose Input / Output Interface (GPIO): Changed the number of GPIO pins to 34 |

Cont’d on next page

Cont’d from previous page

| Date | Version | Release notes |
| --- | --- | --- |
| 2017.06 | V1.5 | - Section CPU and Memory: Changed the power supply range - Section 2.5.2 Power Scheme: Updated the note - Updated Table 5-1 Absolute Maximum Ratings - Table Notes on ESP32 Pin Lists: Changed the drive strength values of the digital output pins in Note 8 - Added the option to subscribe for notifications of documentation changes |
| 2017.05 | V1.4 | - Section Clocks and Timers: Added a note to the frequency of the external crystal oscillator - Section 3 Boot Configurations (used to be named as ”Strapping Pins”): Added a note - Updated Section 4.3 RTC and Low-power Management - Table 5-1 Absolute Maximum Ratings: Changed the maximum driving capability from 12 mA to 80 mA - Table 5-6 Wi-Fi Radio: Changed the input impedance value of 50Ω to output impedance value of 30+j10 Ω - Table Notes on ESP32 Pin Lists: Added a note to No.8 - Table IO_MUX: Deleted GPIO20 |
| 2017.04 | V1.3 | - Added Appendix Notes on ESP32 Pin Lists - Updated Table 5-6 Wi-Fi Radio - Updated Figure 2-2 ESP32 Pin Layout (QFN 5*5, Top View) |
| 2017.03 | V1.2 | - Table 2-1 Pin Overview: Added a note - Section 4.1.2 Internal Memory: Updated the note |
| 2017.02 | V1.1 | - Added Section 1 ESP32 Series Comparison - Updated Section MCU and Advanced Features - Updated Section Block Diagram - Updated Section 2 Pins - Updated Section CPU and Memory - Updated Section 4.2.3 Audio PLL Clock - Updated Section 5.1 Absolute Maximum Ratings - Updated Section 6 Packaging - Updated Section Related Documentation and Resources |
| 2016.08 | V1.0 | First release. |

Disclaimer and Copyright Notice

Information in this document, including URL references, is subject to change without notice.

ALL THIRD PARTY’S INFORMATION IN THIS DOCUMENT IS PROVIDED AS IS WITH NO WARRANTIES TO ITS AUTHENTICITY AND ACCURACY.

NO WARRANTY IS PROVIDED TO THIS DOCUMENT FOR ITS MERCHANTABILITY, NON-INFRINGEMENT, FITNESS FOR ANY PARTICULAR PURPOSE, NOR DOES ANY WARRANTY OTHERWISE ARISING OUT OF ANY PROPOSAL, SPECIFICATION OR SAMPLE.

All liability, including liability for infringement of any proprietary rights, relating to use of information in this document is disclaimed. No licenses express or implied, by estoppel or otherwise, to any intellectual property rights are granted herein.

The Wi-Fi Alliance Member logo is a trademark of the Wi-Fi Alliance. The Bluetooth logo is a registered trademark of Bluetooth SIG.

All trade names, trademarks and registered trademarks mentioned in this document are property of their respective owners, and are hereby acknowledged.

Copyright © 2025 Espressif Systems (Shanghai) Co., Ltd. All rights reserved. www.espressif.com